## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Drawing(4234)/(FE302)[Old/Revised]

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Structural Engineers Handbook SP:6(1) & IS 800 is permissible.

5) All dimensions in the figures are in ‘mm’ unless specified.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between fabrication drawing and design drawing. |  |
| b) | Give the values of ‘a’, ‘b’ and ‘tw’ of the following rolled steel sections: (i) ISMC 175 @19.1 Kg/m (ii) ISNT 30 @ 1.4 Kg/m (iii) ISBA 300 @ 52.6 Kg/m. |  |
| c) | Sketch the following rivet heads: (i) Button (ii) Conical (iii) Flat. |  |
| d) | Calculate the end distance and distance between two consecutive rivets when a rivet of diameter 14mm is used for joining an angle to truss. |  |
| e) | Determine the weight of steel plate of size 231mm x 845mm x 16mm thick. |  |
| f) | Sketch a typical ridge joint. |  |
| g) | What is the purpose of pipelines? List different piping materials. |  |
| Q.No.2. | Draw the fabrication drawing of gusset plate at ‘A’ and ‘B’ and determine the exact length of member A-B of the portion of truss shown in fig.1. Note: (i) All members are ISA 5050 x 4.0mm (ii) Snap headed rivets of 12 mm diameter are used (iii) Gusset plate thickness is 6.0mm. | (12) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a double riveted single strap Bolt joint (chain), joining two plates of thickness 8.00mm and 10.0mm. Indicate dimensions clearly. |  |
| b) | Interprete the weld symbolic representation shown in fig.2. |  |
| c) | Sketch a column splicing for joining bottom column ISHB 200 @ 37.3 Kg/m and top column ISWB 175 @ 22.1 Kg/m. Indicate dimensions clearly. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | The line diagram of structure made up of rolled steel section ISJC 125 @ 7.9 Kg/m is shown in fig.3. Draw to suitable scale the orthographic views (FV, TV, SV) of the structure and also draw the isometric view at location ‘X’. | (8) |
| b) | Make a Bill of Material (B.O.M.) of the structure shown in fig.3, QNo.4a. | (4) |
|  | ### OR |  |
| b) | State the importance of base plate in the fabrication of truss. | (4) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Draw the orthographic views and developed view of the portion of pipeline whose pictorial view is shown in fig.4. | (8) |
| b) | Explain the Flanged pipe joint with sketch. | (4) |
|  | ### OR |  |
| b) | Sketch the Bolted base connections (welded). | (4) |
|  | -2- |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Define the terms beam, column and girder. |  |
| b) | Draw to scale the cross-section of ISJB 175 @ 8.1 Kg/m. |  |
| c) | Explain the symbolic representation of welded joints. |  |
| d) | Draw the sketches of various pipe bends. |  |