## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the scope of surveying for the Civil Engineers? |  |
| b) | Define the following terms: - Baseline ii) Tie line iii) Check line. |  |
| c) | Draw the conventional signs used in survey maps in respect of following: - Marshy land ii) Road embankment iii) Culvert. |  |
| d) | Differentiate between ‘Whole circle Bearing’ and ‘ Quadrantal bearing’. |  |
| e) | State the specific use of following minor instruments: - Plainimeter ii) Abney level iii) Box sextant |  |
| f) | Define the following terms: - Reduced level ii) Height of collimation iii) Bearing of a line |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch and explain the instruments used in measuring distances for a housing project. |  |
| b) | Explain direct and indirect ranging with neat sketches. |  |
| c) | Mention the three types of obstacles likely to be encountered while doing a project survey. How will you overcome them? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between the following with neat sketches: - Fore Bearing and Back bearing - Triangulation and Traversing |  |
| b) | Draw a neat sketch of a Prismatic compass and state the uses of each component. |  |
| c) | Sketch to a convenient scale the cross staff survey ABCDEFGH from the field data given below: E Δ. 400. F 60; 300. 270; 100 D. G 80; 220. 200; 120 C. H 100; 150. 80; 40 B. A; Δ. Calculate the area in square kilometer assuming that all readings are in metres. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following table will give the observed FB and BB of compass closed traverse where local attraction was suspected. Sr.No.; Line; FB; BB. 1; AB; 60 30'; 240 30'. 2; BC; 99 15'; 278 0'. 3; CD; 38 45'; 221 30'. 4; DE; 321 15'; 139 45'. Determine the corrected Fore and Back bearings. At which stations do you suspect local attraction? |  |
| b) | Explain the following terms used in levelling: - Back sight ii) Foresight iii) Datum iv) Change point v) Line of Collimation vi) Axis of telescope. |  |
| c) | What are the two methods used in reduction of levels? Explain each with examples and also mention their suitability. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the temporary adjustments of a dumpy level? Explain in brief. |  |
| b) | The following consecutive readings were taken with a Dumpy level and a 4m staff on a continuously sloping ground at a common interval of 30m. 0.780, 1.535, 1.955, 2.430, 2.985, 3.480, 1.155, 1.960, 2.365, 3.640, 0.935, 1.045, 1.630, 2.545 The R.L of first point was 180.750. Rule out a page of level field book and enter the above readings. Calculate the R.L.s of all the points and also gradient of the line joining the first and last point. |  |
| c) | State the classification of levelling and explain any two with neat sketches. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Types of Bench marks |  |
| b) | Study and use of Auto level |  |
| c) | Types of levelling staff |  |
| d) | Sources of errors in levelling |  |
| e) | Permanent adjustment of a dumpy level |  |

. ***********