## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(4331)/(SH301) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Passenger vessel |  |
| b) | Keel and Camber |  |
| c) | Archimedes principle |  |
| d) | Half breadth plan |  |
| e) | Metacentric height |  |
| f) | MCT |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the salient features of: i) Bulk carrier ii) Dredger. |  |
| b) | A ship has 300 tonne of cargo in the hold, 24m forward of midships. The displacement of the vessel is 6000 tonnes and its centre of gravity is 1.2m forward of midships. Find the new position of the centre of gravity if this cargo is moved to an after hold, 40m from midships. |  |
| c) | Find the area of the waterplane of a ship 36m long, 6m beam, which has a coefficient of water plane area of 0.8. Also find the TPC of the vessel. |  |
| Q.No.3. | The areas of a ship’s water-planes commencing from the load water plane and spaced at equidistant intervals down to the bottom are: 2500, 2000, 1850, 1550, 1250, 900 and 800 m2 respectively. The load draft is 6m. Find the load displacement in salt water and the height of the centre of bouyancy above the keel. | (12) |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A box shaped vessel 60m x 10m x 3m is floating upright in fresh water on an even keel at 2m draft. When a mass of 15 tonnes is moved 6m transversely across the deck, a 7m plumb line is deflected 20cm. Find the ship’s KG. (Only for students of revised scheme) |  |
| a) | Explain frees surface effect in ships (Only for students of old scheme) |  |
| b) | A box shaped vessel is 24m x 5m x 5m and floats on an even keel at 3m draft. If KG=2m, calculate the metacentric height. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A ship of 8000 tonnes displacement has KG =3.75m and KM=5.5m. A double bottom tank 16m x 16m x 1m is subdivided at the centreline and is full of salt water ballast. Find the new GM if this tank is pumped out until it is half empty. |  |
| b) | The half ordinates of a water plane area 180m long are as follows: Section; AP; 1; 2; 3; 4; 5; 6. ½ ord; 0; 8.0; 10.5; 12.5; 11.0; 3.0; 1.0. Calculate i) TPC ii) LCF iii) Second moment of area of waterplane about centerline. | ……2/- |
| -2- |  |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the following terms: - Block co-efficient ii) TPC iii) LCB |  |
| b) | A ship has length 128m, breadth=17.7m and draught =7.9m. Areas of the immersed section at various stations are: |  |
| Stations; AP; 1/2; 1; 2; 3; 4; 5; 6; 7; 8; 9; 9 1/2; FP. Area(m2); 4.00; 26.03; 49.32; 94.53; 121.93; 135.63; 137; 135.63; 128.78; 101.38; 45.21; 17.81; 0. |
|  | Determine displacement and midship area coefficient. |  |

***********