## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engineering

Subject: Garage Equipments(AT401) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How would you remove a piece of a broken bolt? |  |
| b) | What is meant by ‘Calibration’ of an instrument? |  |
| c) | Give an example of how and where a Vernier depth gauge can be used. |  |
| d) | What are the parameters normally checked in a Battery tester? |  |
| e) | When would you use a ‘Mallet’ instead of a regular hammer? |  |
| f) | Give an example of where a ‘Pulley block’ is preferred over a Trolley jack? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the procedure for checking the compression of an engine using a ‘compression gauge’. |  |
| b) | Draw a neat sketch of a Cylinder Dial gauge and explain how it is used in the inspection of an engine cylinder. |  |
| c) | What is a ‘Combination set’? What are the various measurements that can be carried out using a Combination set? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how you would use a ‘Tap’ to cut threads on a shaft. Use a suitable sketch. |  |
| b) | Draw neat sketches of any 2 types of Pliers. |  |
| c) | Draw diagrams of any two types of handles used with socket spanners and where would you recommended to use them? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a ‘Toggle Jack’ and lable its parts. Also mention the precautions to be taken while lifting a vehicle using a toggle jack. |  |
| b) | Explain the function and working principle of a Spark Plug Tester. |  |
| c) | Explain the procedure involved in repairing a punctured tube of a vehicle tyre. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how you would use an ‘Inside Micrometer’ to check a cylinder bore. |  |
| b) | How would you classify ‘Calipers’? Show sketches of any two of them. |  |
| c) | Draw a block diagram of a steam car washing system and explain its working. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a crank shaft grinding machine and lable its parts. |  |
| b) | With the help of a neat sketch, pictorially differentiate between a 2-post & 4-post lift. Where would you recommend the use of each one? |  |
| c) | Write a short note on the ‘Manual Spray Painting’ procedure in painting a used vehicle. |  |

***********