## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Architectural Engg.

Subject: Estimation & Costing(4529)/(AE503) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Why is it essential for Architect to possess knowledge of estimation & costing? | (3) |
| b) | Explain the importance of intermediate measurement in billing. | (4) |
| c) | Explain how rate analysis is affected by labour factor. | (4) |
| d) | Explain the importance of schedule of rates for an estimator. | (4) |
| Q.No.2. | Prepare schedule of quantities for electrical points as given below without cost calculation. Refer plan in fig.3. (Assume location of points if found necessary) S.No; Location; Light pt; Tube No; Fan; 5amp plug; Power plug; Bell. 1; Hall; 1; 1; 1; 1; -; 1. 2; Kitchen; 1; 1; 1; 1; 1; -. 3; Toilet; 1; -; 1 Exhaust; 1; 1; -. 4; Veranda; 1; -; -; -; -; -. | (12) |
|  | ### OR |  |
| Q.No.2. | #### Answer the following Questions: | (12) |
| a) | Write specifications for: i) RCC isolated footing ii) Vitrified tile flooring. |  |
| b) | Briefly explain the term ‘schedule of rates’. |  |
| Q.No.3. | Calculate quantities of any two of the following items of works and prepare bill with reference to fig.3. i) Internal cement Plaster (1:4) 12mm thick ii) Laterite stone masonry in superstructure iii) Internal flooring including Dadoing in toilet | (12) |
| Q.No.4. | #### Answer the following Questions: |  |
| a) | Calculate masonry in superstructure for shed in fig.1 using centre line method only. | (4) |
| b) | Prepare rate analysis for any one item of works: i) R.C.C in beam (4m3) ii) Internal painting in two coats for an old building (surface area 100m2) | (8) |
|  | (only for students of Revised scheme) |  |
| Q.No.5. | Answer the following Questions: | (4) |
| a) | Write a brief note on various types of estimates. |  |
|  | ### OR |  |
| a) | What is the advantage of item rate estimate over other types of estimates? | (4) |
| b) | List the components of Measurement sheet and Abstract sheet. Briely explain the importance of each component. | (4) |
| c) | Specify the units followed for below mentioned items of works: i) Back filling in foundation trenches 1.5m wide ii) Half brick masonry in superstructure iii) External painting iv) Electrical work. | (4) |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Calculate the quantities for following item of works: (any two) | (12) |
|  | i) Masonry in steps as shown in fig.1 |  |
|  | ii) RCC lintel over 1m wide opening (Assume size of lintel) |  |
|  | iii) Masonry for compound wall (length 20m), refer fig.2 |  |
|  | (only for students of Old scheme) |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write briefly on various types of estimates. |  |
| b) | What is the advantage of item rate estimate over other types of estimates? |  |
| c) | State the advantage of preparing approximate estimate. |  |
| d) | Write a short note on ‘Measurement book’ on the site. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Calculate the quantities of following item of works: (any one) | (4) |
|  | i) Masonry in steps as shown in fig.1 |  |
|  | ii) RCC lintel over 1m opening (Assume lintel size) |  |
| b) | Discuss on how rates are fixed per unit of item. | (8) |
|  | ### OR |  |
| b) | Discuss on factors that affect the rates of items. | (8) |