## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(ME614) [Rev.]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List any 3 classifications of automobiles based on body style and describe each of them in brief. |  |
| b) | Distinguish between port injection and throttle-body injection, in gasoline fuel injection systems. |  |
| c) | What is battery self-discharge? |  |
| d) | Discuss in brief the irreversibility of a steering gear. |  |
| e) | Draw a neat sketch of the fluid flywheel characteristic and suggest how engine should be operated. |  |
| f) | List the functions of suspension system of an automobile. |  |
| g) | State the advantages and disadvantages of hydraulic brakes. |  |
| h) | Draw neat sketch of a ‘piston ring compressor’ tool. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the power flow in a 2WD vehicle. |  |
| b) | Distinguish between conventional chassis frame and monologue design. |  |
| c) | List the various types of loads on a chassis frame. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is clutch pedal free play? Why is it kept? What is clutch riding? What are its effects? |  |
| b) | With the help of a neat sketch, explain the working of a torque convertor. |  |
| c) | With the help of a neat sketch, explain the principle of working of a simple automatic transmission. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how fuel injection timing is done in a Jerk Pump system diesel engine. |  |
| b) | List the advantages of MPFI engine over carburetion engine and explain in brief. |  |
| c) | Explain with the help of a neat circuit diagram, the battery and coil ignition system of a petrol engine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a thermostatic expansion valve in the air conditioning system of an automobile. |  |
| b) | With the help of a neat sketch, explain the working of a thermostatic cycling switch in the air conditioning system of an automobile. |  |
| c) | With the help of a neat block diagram, explain the working of pneumatic brake system of an automobile. |  |
| d) | With the help of a neat block diagram, explain the engine management system of an MPFI engine. | ……..2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of steering linkage for rigid axle suspension vehicle. |  |
| b) | With the help of a neat sketch, explain the working of integral type hydraulic power steering. |  |
| c) | With the help of a neat sketch, explain the working of double wishbone type independent suspension. |  |
| d) | With the help of a neat sketch, describe various types of torque wrenches. |  |

***********