## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Repair Engineering(5334)/(SH602) [Old/Rev.]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Docking plan and its significance |  |
| b) | Thickness gauging and limits of acceptance |  |
| c) | Rudder drop measurements |  |
| d) | Testing water tightness of closing appliances |  |
| e) | Boiler mountings |  |
| f) | Survey of fire fighting appliances |  |
| g) | Work done certificate |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the procedure of docking a vessel in a Graving Dock. |  |
| b) | Which are the documents/plans used as reference, while preparing a survey programme? |  |
| c) | What tests are carried out after completion of repairs on repaired areas and compartments of hull? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the methods used for detecting cracks during survey of a propeller shaft? Which part of the shaft is subjected to crack detection? Give reasons. |  |
| b) | What are the items specially checked during survey of lifeboat davits? |  |
| c) | During load line survey preparation, how are watertight doors, air pipes and ventilators tested and repaired? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different regulations concerning the ship’s painting? State the reasons for providing insulation on-board ship. |  |
| b) | Describe the procedure of overhauling and calibration of a fuel injector. |  |
| c) | Explain the checks carried out on a Gear pump opened for survey. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why are the periodic safety drills carried out on-board ships? What are the two main safety drills carried out? Explain in brief how any one of these drills is carried out. |  |
| b) | If a vessel has to undergo a survey, how the defect lists are prepared? What is job order? What all points are considered, while billing the client? |  |
| c) | Describe maintenance and testing of Fire mains and Hydrants on-board ship. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the procedure for removal and maintenance checks carried out on piston rings of a main diesel engine. | ….2/- |
|  | -2- |  |
| b) | What safety precautions are to be considered for repair work inside a tank including ventilation and lighting? |  |
| c) | Describe the inspection and maintenance carried out on a fresh water Generator. |  |
| Q.No.7. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Services provided to ships under repairs |  |
| b) | Survey of bilge keels |  |
| c) | Standards of surface finish |  |
| d) | Safety for confined spaces works |  |

***********