## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg./A/SB

Subject: Strength of Materials(ME401) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define stress and strain. |  |
| b) | Mention the different types of beams with sketches. |  |
| c) | What is radius of gyration? Explain. |  |
| d) | State the parallel axis theorem. |  |
| e) | What is moment of resistance as applied to bending? |  |
| f) | With the help of a neat sketch, show the shear stress distribution over a section of a circular shaft subjected to torsion. |  |
| g) | Fig.1 shows the shear force diagram for a simply supported beam. Draw the load distribution diagram on the beam. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the force ‘P’ acting at ‘C’ on the bar shown in Fig.2. Find the extension of the bar if E = 2 x 105 MPa. |  |
| b) | A round bar 40mm diameter is subjected to an axial pull of 80 KN and reduction in diameter is found to be 0.00775mm. Find Poisson’s ratio and Young’s modulus for the material of the bar. Take value of modulus of rigidity as 40 KN/mm2. |  |
| c) | A bar of 400mm2 cross-sectional area shown in fig.3 is attached to a rigid support at one end and the other end is at a distance of 0.3mm from the other rigid support at room temperature. i) What is the maximum temperature rise, which will not produce any stress in the bar? ii) Find the stress induced in the bar when the temperature rise is 40C. Take E = 1.2 x 105 N/mm2 and  = 17.5 x 10-6/C |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A point in a strained material is subjected to two mutually perpendicular tensile stresses of 200 N/mm2 and 100 N/mm2 as shown in fig.4. Determine the intensities of normal, tangential and resultant stresses on a plane inclined at 30 anticlockwise to the axis of the minor stress. | …….2/- |
|  | -2- |  |
| b) | The state of stress at a point in a strained material is shown in fig.5. Determine analytically the principal stresses and maximum shear stress. Also determine the directions of the principal planes. |  |
| c) | Fig.6 shows a T-section. Determine the moment of inertia about its horizontal centroidal axis. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cantilever beam has point loads at B, C and D as shown in fig.7. Draw the shear force and bending moment diagrams for the cantilever. |  |
| b) | A simply supported beam has a uniformly distributed load between B and C as shown in fig.8. Draw the shear force and bending moment diagrams for the beam. |  |
| c) | A Cantilever beam shown in fig.9 has point loads at B and C and a udl between B and C. Draw the shear force and bending moment diagrams for the cantilever. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam of hollow rectangular section is 40mm wide and 150mm deep on the outside and 10mm thick on all sides. It is simply supported over a span of 5m as shown in fig.10. If the udl on the beam for the entire span is 1500 N/m, determine the maximum bending stress in the beam. |  |
| b) | A cantilever of circular section is 50mm in diameter and 3.5m long. Determine the magnitude of udl it can carry if the maximum stress developed in the cantilever is 80MPa. |  |
| c) | A rectangular column 200mm wide and 150mm thick carries a vertical load of 120KN as shown in fig.11. Determine the maximum and minimum intensities of stress in the section. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A solid shaft transmits 560KW at 300 rpm. The permissible shear stress in the shaft is 60 N/mm2. Determine the diameter of the shaft. |  |
| b) | A hollow shaft has outer and inner diameters 200mm and 150mm respectively. If the maximum shear stress is not to exceed 50 N/mm2, find the maximum power than can be transmitted at 200 rpm. |  |
| c) | Determine the moments of inertia about the horizontal and vertical centroidal axes (X-X and Y-Y) for the I – section shown in fig.12. |  |