## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Process Control(4203) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Draw the block diagram of a process control loop & briefly describe the function of each block. | (4) |
| b) | Write short notes on process objectives. | (4) |
| c) | Define process load. What is process lag? | (2) |
| d) | Write the process equation for a PID controller. | (1.5) |
| e) | What is a portable data logger? | (1) |
| f) | Name the three basic types of disturbances in a control system. | (1.5) |
| g) | Write the expression for flow rate of a liquid through a control valve. | (1) |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write an event sequence description for the process shown in figure 1. Objective of the process is to fill bottles. |  |
| b) | Draw the ladder diagram symbols for: i) NO and NC relay ii) NO and NC Push button switch iii) NO and NC Temperature limit switch iv) Motor v) Solenoid vi) Light bulb. |  |
| c) | What are input modules in a programmable setup? Describe the interconnection of input module to a processor with an example. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the following control system parameters: Error, Variable range, Control parameter range. |  |
| b) | What is floating control mode? Describe single speed and multiple speed control action. |  |
| c) | Describe briefly any two composite controller modes. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe briefly the implementation of Integral and Proportional Derivative (PD) Controller mode using Opamp. |  |
| b) | With a neat diagram describe implementation of PID controller using Opamps. |  |
| c) | Describe briefly with neat diagrams implementation of Derivative and Proportional Integral (PI) controller modes using pneumatic controllers. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain fixed data loggers with a block diagram. |  |
| b) | Describe multiple variable alarm in process control with an example. |  |
| c) | What is a single variable loop? Briefly discuss its types. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is stability, minimum elevation and minimum duration? Briefly describe. |  |
| b) | Draw and explain the functional block diagram of final control operation. |  |
| c) | Describe briefly either Analog or Digital electrical signal conversion systems. |  |