## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT

Subject: Construction Materials(4003) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the three ways of classification of rocks? Name 2 types of rocks in each class. |  |
| b) | Enlist any 6 characteristics of good tiles (clay product). |  |
| c) | What are the porcelain products? Where they are used? |  |
| d) | Specify any 3 uses of lime in Civil Engineering works. |  |
| e) | Enlist any 6 advantage of use of wood in construction of buildings. |  |
| f) | Enlist any three methods of artificial seasoning of wood. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enlist any three characteristics of good stones used in construction and explain in brief the tests to check these qualities. |  |
| b) | Write short note on manufacturing of bricks point wise in at least six points. |  |
| c) | Explain in brief the test on compression strength and water absorption for building bricks. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Name any 6 clay products and mention where they are used in construction. |  |
| b) | i) Specify any three uses of lime. ii) Name any three types of cements. |  |
| c) | List out the four chemical compounds formed during setting of cement. Indicate their range of percentage and functions of each. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief: fineness, soundness and setting time of cement. |  |
| b) | i) Explain in brief the various sources of sand. ii) List out any 6 characteristics of good sand. |  |
| c) | Write short notes on the following, specifying their specific use: i) Lime mortar ii) Cement mortar iii) Surkhi mortar. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain point wise the natural method of seasoning of wood, with sketches. |  |
| b) | Name any 6 types of special glasses. |  |
| c) | Specify any 6 properties of plastic which make it useful in construction industry. |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Specify any 3 uses of Asbestos in Engineering works. |  |
| b) | Name any 3 forms of asphalt and any 3 uses. |  |
| c) | Name any three types of glues and use of glue in buildings. |  |
| d) | Specify any 3 uses of rubber in construction. |  |
| e) | List out uses of aluminum in buildings. |  |
| f) | Enlist any 6 properties of good stones for use in construction. |  |

***********