## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./EC

Subject: Programmable Logic Controller(EN611) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List three features of a PLC. |  |
| b) | List the types of input and output modules of a PLC. |  |
| c) | Explain what is a ladder diagram and a functional block diagram. |  |
| d) | Explain how a combination up-down counter can be constructed using up counter and down counter blocks. |  |
| e) | Explain how a Master Control Relay works. |  |
| f) | Explain the force method of testing inputs and outputs. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a block diagram of a PLC and explain the components of a PLC. |  |
| b) | With the help of a diagram, explain sinking output and sourcing output and the difference between the two. |  |
| c) | Define a PLC. List three advantages of PLC and any two applications of a PLC. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Draw a ladder diagram for the following Boolean expression: A⋅B+C+D |  |
|  | ii) Explain the following basic PLC instructions: 1) Examine if closed 2) Output Energise 3) Output latch |  |
| b) | Explain the major features of the IEC 1131-3 standard. |  |
| c) | i) Draw ladder diagrams to represent the following logic functions: - NOR 2) AND 3) NAND |  |
|  | ii) Draw a ladder diagram that will energise an output when a START button is pressed. The output should stay energized when the START button is released and should de-energise when a STOP button is pressed. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a ladder diagram for a capsule–manufacturing process. When the START button is pressed a conveyor belt carries bottles to be filled with 30 capsules each at the Filling Station. Once filled, the conveyor moves the bottles to be capped at the Capping Station. The entire process should be stopped when the STOP button is pressed. |  |
| b) | Draw and explain the function block of an On-delay timer. |  |
| c) | i) List the types of timers in PLCs. |  |
|  | ii) Explain the limit test data comparison instruction. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe in brief: - Internal relays - Battery backed relays |  |
|  |  | …..2/- |
|  | -2- |  |
| b) | Explain the Bit Shift Right instruction. Draw the functional block for this instruction and explain the parameters associated with this block. |  |
| c) | Draw and explain the function blocks associated with subroutines. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the following fault detection techniques in PLC: - Timing checks ii) Last output set |  |
| b) | Give a brief overview of what a PLC system documentation should include. |  |
| c) | i) Explain in brief emergency stop relays. |  |
|  | ii) Give an example of an application of shift registers in PLC. |  |

***********