## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mining Engg.

Subject: Mine Surveying(MN611) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the duties & responsibilities of Mine Surveyor? |  |
| b) | What do you understand by limit of error? |  |
| c) | Give the application of Unmanned Aerial Vehicle in surveying. |  |
| d) | How do you set up control points in opencast survey? |  |
| e) | What are the purposes of stope surveying? |  |
| f) | Draw any six conventional symbols used for preparation of mine plans. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the classification of errors. |  |
| b) | What are the steps taken in preparation of plans & sections? |  |
| c) | i) What are the components of GIS? ii) How do you control direction & gradient in tunnels? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following offsets are taken from a survey line to a curved boundary line: Distance (m); 0; 5; 10; 15; 20; 30; 40; 60; 80. Offset (m); 2.80; 3.75; 4.60; 5.20; 6.30; 4.90; 5.60; 4.20; 2.20. Find the area between the survey line, the curved boundary line and the first and the last offsets by i) the Trapezoidal rule and ii) Simpson’s rule. |  |
| b) | Calculate the weight of coal in a heap, which occupies an area measuring 27m x 8m and is 6m high. The sides of the heap slope at 45 and the coal density is 1.3 T/m3. |  |
| c) | i) What precautions are taken during measurement of coal stock on uneven ground? ii) A mine roadway is constructed with 3.6m x 3m arches and is 450m in length. Calculate the volume of this length of the road. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two seams of coal are separated by 42m (vertical) of strata which dip at angle of 20 to the horizontal. Calculate the length of the cross-measure drift driven to connect them, if it dips 1 in 2 in the same direction as the seams. |  |
| b) | Explain correlation by Weisbach triangle method with a neat sketch. |  |
| c) | Enumerate the factors affecting selection of a method of stope surveying. Give a brief note on surveying of shrinkage stopes. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure of setting up & measurement of EOM. |  |
| b) | What are the advantages & disadvantages of total station? Explain the principle of operation. | ….2/- |
|  | -2- |  |
| c) | i) Explain the principle of GPS. ii) What do you understand about DGPS? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the principle of operation of Gyrotheodolite. |  |
| b) | How do you predict strata movement in mines? |  |
| c) | Explain curve setting by chord & angle method. |  |
| d) | Explain any one method of enlarging of plans. |  |

***********