## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT/M

### Subject: Hydraulics(4025)/(CE405) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the specific weight, specific density and specific gravity of 1 litre liquid which weighs 10N. |  |
| b) | What are the different pressure measuring devices? |  |
| c) | Define i) Total Pressure Force ii) Centre of Pressure. |  |
| d) | State ‘Bernoulli’s Theorem’ and write its equation for real fluid. |  |
| e) | What are the advantages of Triangular Notch over Rectangular Notch? |  |
| f) | Explain in brief ‘Hydraulic Jump’. |  |
| g) | What is Mouthpiece? What role it plays if connected to tank containing water? What is meant by ‘Venacontracta’? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State ‘Pascal law’ for fluid. Find the depth below oil surface where intensity of pressure is 20KPa. Specific gravity of oil is 0.9. |  |
| b) | Find Total Pressure Force and Centre of Pressure of triangular plate of base 2m and height 3m immersed in a liquid with its plane making an angle of 60 with the free surface of water. The base of triangular plate is parallel to water surface and coincides with free surface. |  |
| c) | Explain with a neat sketch ‘Differential U-Tube Manometer’. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 30x20cm Venturimeter is inserted in a vertical pipe carrying liquid with specific gravity 1.2. A ‘Differential U-Tube Manometer’ connected to the inlet and the throat gives mercury level difference of 20cm. Find the discharge. Take Cd=0.98 |  |
| b) | Water is flowing through a tapering pipe having diameter 300mm & 150mm at section 1 and 2 respectively. The discharge is 0.04m3/s. Section 1 is 10m above datum and section 2 is 6m above datum. Find intensity of pressure of section 2 if that of section 1 is 400KN/m2. |  |
| c) | Water is discharged at a rate of 0.098m3/s through 12cm diameter orifice under a constant Head of 10m. A point on a jet, measured from venacontracta of the jet has co-ordinates 4.5m horizontal and 0.54m vertical. Find coefficients Cd, Cv, Cc. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A tank has two identical orifice in one of its vertical sides. The upper orifice is 3m below the water surface and the lower one is 4m below the water surface. The coefficient of velocity for each orifice is 0.95. Find the point of intersection of two jets. |  |
| b) | Find the discharge over Cipolletti weir of length 2m when the head over the weir is 1.4m. Take Cd=0.62 |  |
| c) | Find discharge over triangular notch of angle 60 when head over the V-notch is 30cm. Take Cd=0.6 | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a sudden enlargement of water main from 250mm to 500mm diameter, the hydraulic gradient rises by 12mm. Calculate the rate of flow. |  |
| b) | The piping system consists of 3 pipes arranged in series. The length of the pipes are 1200m, 800m, 400m and diameters are 75cm, 50cm, 35cm respectively. Determine an equivalent diameter for the pipe 2000m long. |  |
| c) | Find the rate of flow for Rectangular open channel 8m wide and 2.5m deep having bed slope 1 in 1200. Take Chezy’s constant C=55. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the bed slope of Trapezoidal channel bed width 4m and depth of water 2m. The side slopes are 2 horizontal and 3 vertical. The discharge through channel is 25m3/sec. Take Manning’s N=0.03 in the Manning Formula C=1/N m1/6. |  |
| b) | Distinguish clearly between Centrifugal pump and Reciprocating pump from the point of view of selection and principle of working. |  |
| c) | Write short notes on any two: i) Most economic section for the channel ii) Major and Minor losses in pipe iii) Bourden Pressure Gauge |  |

***********