## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Metrology & Q.C.(4047)/Engineering  Metrology & Q.C.(ME406) [Old/Rev.]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State Taylor’s principle of gauge design. |  |
| b) | How do we check: i) squareness of a rectangular block ii) parallelism of an axis to another axis formed by intersection of two inclined planes? |  |
| c) | What is the purpose of having quality control in mass production? |  |
| d) | Discuss statistical quality control as a tool for quality check. |  |
| e) | What are attributes & variables? Give one example of each. |  |
| f) | How do we measure effective diameter of a screw thread using micrometer? |  |
| g) | Draw and label a sketch of an Optical Bevel protractor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | (i) Define Metrology. (ii)Explain need for inspection in industries for quality control. |  |
| b) | (i) Define Line and end standards. (ii) With the help of a sketch, explain the main parts of a sine bar. |  |
| c) | Explain how Angle Dekkor can be used for checking the accuracy of a V-block. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the characteristics of various fits? Give example of each type. |  |
| b) | Work out the ‘General’ type gauge tolerances and allowances for both ‘Go’ & ‘Not-Go’ gauge if hole tolerance is worked out to be 0.0297mm & for shaft 0.0209mm. |  |
| c) | Give broad classification of comparators. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly discuss the procedure for determining the flatness of a surface using a sprit level. |  |
| b) | Explain the procedure of measuring major diameter of external threads using bench micrometer. |  |
| c) | What are the objectives of a quality control programme? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | By studying the data given and computing the limits of control, sketch a p-chart for a factory data given as: |  |
| Week; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. Fraction defectives; 0.15; 0.12; 0.09; 0.09; 0.18; 0.2; 0.13; 0.14; 0.11; 0.10. |
| b) | Draw an operating characteristic (OC) curve, and explain the various terms involved. |  |
| c) | Explain Double sampling plan. | …….2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Symbols used in indicating surface roughness on drawings |  |
| b) | Effects of thread pitch errors |  |
| c) | Angle gauges |  |
| d) | Relationship between cost and value of quality of product |  |
| e) | Requirements of a good dial indicator |  |

***********