## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(4026)/(CE504) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Derive an expression for BM for a fixed beam carrying point load at centre. |  |
| b) | Two timber joists 120mm wide and 300mm deep are reinforced by a steel plate 12mm thick and 250mm deep placed symmetrically and clamped in between them. Determine the moment of resistance MR of the beam if the safe stress in timber is 7.5N/mm2. Take Es = 16Ew. |  |
| c) | A point in a strained material is subjected to two mutually perpendicular tensile stresses of 150N/mm2 and 75 N/mm2. Using Mohr’s circle, locate the plane across which the Resultant stress has maximum obliquity. |  |
| d) | Derive an expression for slope at free end and deflection at free end for a cantilever beam carrying concentrated load ‘W’ at free end. |  |
| e) | Determine the limit of eccentricity for the following section: - Hollow Rectangular section - Hollow circular section |  |
| Q.No.2. | Answer any two of the following Questions: | 2x7.5=15 |
| a) | A 4m x 3m column has to transmit a load of 250KN, the load line is eccentric being 5cm above xx axis and 2cm to the right of yy axis. Determine the stress at four corners of the column. |  |
| b) | A simply supported beam carries two point loads of 20KN and 30KN at a distance of 1m and 3m from left support respectively. Calculate the slope at left end and deflection under the loads. Take E = 2x105 N/mm2, and I = 7.5x107 mm4. Use Macaulay’s method. |  |
| c) | A Timber beam 100mm wide and 160mm deep is to be strengthened. Two steel plates 100mm x 12mm and 100mm x 6mm are adequately secured to it, the thicker plate to the top surface and thinner to the lower surface. If the maximum permissible stress in steel is 140 N/mm2 and modular ratio is 20, calculate the MR of the strengthened section. Assume that the timber will not be overstressed. |  |
| Q.No.3. | Answer any two of the following Questions: | 2x7.5=15 |
| a) | A material is subjected to normal stress of 70 N/mm2 on xx axis and 50 N/mm2 on yy axis, both tensile. Also, shear stress of 40 N/mm2 across both the planes carrying normal stresses. By Mohr’s circle method determine the principal stresses and maximum shear stress. |  |
| b) | The principal stresses at a point in a material are 180 N/mm2 (tensile) and 85 N/mm2 (compressive). Determine normal, Resultant and tangential stress on a plane inclined at 60 to the axis of major stress. Solve analytically. |  |
|  |  | …..2/- |
|  | -2- |  |
| c) | A fixed beam having a span of 6m carries a uniformly distributed load of 15 KN/m over its entire span in addition to a point load of 30KN at centre. Analyse the beam and draw SFD and BMD. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 9 = 18 |
| a) | A continuous Beam ABC is fixed at A and simply supported at B. BC is overhand, AB = 4.5m and BC = 2m. A udl of 25 KN/m is acting on the entire span ABC and concentrated load of 45KN acts at free end C. Take EI = 10x1011 KN/mm2 for entire beam. Analyse the beam using moment distribution method. Draw SFD and BMD. |  |
| b) | A portal frame is fixed at A and D such that AB = CD = 6m and span BC = 8m. BC carried a point load of 50KN at centre. Analyse the frame using moment distribution method. Draw BMD. Assume EI = constant. |  |
| c) | A two span continuous beam ABC is simply supported over the entire span, such that AB = 5m and BC = 3m. Span AB carries udl of 10KN/m over its entire span and BC carries point load of 50KN at a distance of 1m from support B. MI for span AB = 2I and BC = 1.5I. Analyse the beam using moment distribution method. Draw BMD and SFD |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | What are the assumptions made in Euler’s theory of columns? Draw neat sketches and write the effective length of columns with different end conditions. (any two) | (4) |
|  | ### OR |  |
| a) | A mild steel tube 25mm internal diameter and 30mm external diameter, length 3m is used as column, one end fixed, the other hinged. Calculate Euler’s crippling load. E= 2x105 N/mm2 | (4) |
| b) | A cast iron column of hollow cylindrical section 5m long, with ends fixed, has to carry an axial load of 350KN. Determine the diameters of the section using a factors of safety of 8. Internal diameter to be 8/10 of the external diameter. Rankines constants are fc = 550N/mm2 and  = 1/1600 | (8) |

***********