## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-I(AT403) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Classify automobiles with respect to the suspension system. |  |
| b) | Explain in brief why a truck with high HP engine has a single–plate clutch while a scooter with much lesser HP has a multiplate clutch. |  |
| c) | State the functions of a gearbox in an automobile. |  |
| d) | Define tractive effort of a vehicle and show how it varies with vehicle speed and in different gears. |  |
| e) | State the advantages of a full-floating axel. |  |
| f) | What is meant by ‘limited slip differential’? |  |
| g) | What is the advantage of a tandem master cylinder? |  |
| h) | Define ‘wheel base’ and ‘wheel track’. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe in brief the various car body styles. |  |
| b) | Draw a neat labelled sketch to show the layout of complete transmission system of a 2WD vehicle. |  |
| c) | Discuss the advantages and disadvantages of framed chassis versus the unitary design. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a single–plate dry clutch. |  |
| b) | What is clutch riding? Is it good or bad? If so, explain why? |  |
| c) | List the various clutch lining materials and indicate their approximate value of coefficient of friction and application. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a constant mesh gearbox. |  |
| b) | With the help of suitable graphs, explain the necessity of gearbox in an automobile. |  |
| c) | With the help of neat sketches, explain the working of a synchronizing unit in a synchromesh gearbox. |  |
| Q.No.5. | With the help of neat sketches, explain the working of any 2 of the following: i) Hooke’s joint ii) Rzeppa joint iii) Transfer case. | (12) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the layout of air brake system of an automobile. |  |
| b) | With the help of a neat sketch, explain the construction and working of a master cylinder. |  |
| c) | Explain the procedure of bleeding of hydraulic brakes, with the help of a neat sketch. |  |

***********