## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Principles of Refrigeration(5046) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the term: i) Industrial refrigeration ii) C.O.P. |  |
| b) | List various methods of refrigeration. |  |
| c) | Why ammonia is used as refrigerant? |  |
| d) | Why P-H, T-S diagrams are plotted? |  |
| e) | What is the function of Throttling device & evaporator? |  |
| f) | List some advantages & disadvantages of Electrolux refrigeration system. |  |
| g) | List the various fields of use of refrigeration system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A heat pump using ammonia as the refrigerant operates between saturation temperatures of 5C and37C. The refrigerant is compressed isentropically from dry saturation and there is 6K of under-cooling in condenser. Calculate i) C.O.P. (heat pump) ii) The mass flow of refrigerant & iii) The actual refrigerating effect. |  |
| b) | What do you understand by term refrigeration capacity? What are the effects of superheating and low compression on refrigeration capacity? |  |
| c) | What are the ideal properties of an refrigerant? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How compressor selection is done? What is the working principle of reciprocating compressor? |  |
| b) | Explain with neat sketch forced convection air cooled condenser. |  |
| c) | Explain the construction of refrigerated freight car. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain two throttling devices. |  |
| b) | Compare the following: i) Dry and flooded type evaporator ii) Bare tube and finned tube type evaporator. |  |
| c) | Write a chemical formulae and determine the designation number for the following refrigerants: i) Mono-chloro-di-fluoro methane ii) Tri-chloro-tri-fluoro ethane iii) Tri-chloro-mono-fluoro methane. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Write down the procedure for refrigeration system servicing. |  |
| b) | Explain the heat balance sheet of evaporator of a water cooler. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Solenoid valve |  |
| b) | Preservation of food |  |
| c) | Automotive air conditioner |  |
| d) | Miscibility of compressor oil in refrigerant |  |

***********