## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Drawing & Calculations(SH603) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Large angle stability |  |
| b) | Permeability |  |
| c) | Section modulus |  |
| d) | Framing system in a ship |  |
| Q.No.2. | The KN values of M.V. ‘Cargo Carrier’ when displacement is 35000 tonnes and KG is 9 m is given below: | (12) |
|  | Heel 5 10 15 20 30 45 60 75 90 () |  |
|  | KN(m) 0.9 2.0 3.2 4.4 6.5 8.75 9.7 9.4 8.4 |  |
|  | Draw the stability curve and find: |  |
|  | i) Range of stability ii) Angle of vanishing stability iii) Max GZ iv) Approximate GM. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | A box shaped vessel 150m long, 20m beam is floating upright in salt water at draught of 6m F and A. The collision bulkhead is situated 8m from the forward. Find the new draughts if the vessel is now bilged forward of the collision bulkhead. | (8) |
| b) | With neat sketch derive the equation to find the change in draught aft and forward when the ship is trim by ‘t’ m by stern. | (4) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Sketch and describe corrugated bulkhead. | (8) |
| b) | Draw a neat sketch to show how an upper end and lower end of corrugated bulkhead is connected to ship structure. | (4) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Draw a neat sketch to show how water tightness is maintained in pipe penetration and longitudinal structural member penetration of transverse bulkhead. | (4) |
| b) | A box shaped vessel 65mx10mx6m is floating upright on an even keel at 4m draught in salt water. GM = 0.6m. Calculate the dynamical stability at 20 heel. | (8) |
| Q.No.6. | Answer the following Questions: |  |
| a) | Describe the load, shear force and bending moment diagram of a ship in still water. Explain how these diagrams are drawn. | (8) |
| b) | Explain how section modulus of a ship is obtained. | (4) |
| Q.No.7. | A box shaped barge of 80m length and light ship weight of 520 tonnes is divided by transverse bulkheads into 4 equal holds. Each hold is loaded uniformly with following cargo: Hold No 1 _______ 540 tonnes Hold No 2 _______ 460 tonnes Hold No 3 _______ 520 tonnes Hold No 4 _______ 520 tonnes | (12) |
|  | Draw the SF and BM diagram indicating maximum bending moment. |  |

***********