## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Common

Subject: Elements of Electrical Engineering(3001) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the specifications for procurement of PVC insulated power cables. |  |
| b) | State the I. E rule for i) Light & fan wiring ii) Power wiring. |  |
| c) | State any two I.E rules relevant to earthing. |  |
| d) | Can a dc motor be started directly on line? Give your choice, giving reasons. |  |
| e) | “A single phase induction motor is not self starting” - say true or false and justify your choice. |  |
| f) | State the advantages of MCB over switch fuse unit. |  |
| g) | What is the function of relay? State their various types which you know. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State at least six advantages and four disadvantages of overhead distribution system over underground distribution system. |  |
| b) | Enumerate the various factors on which current carrying capacity of a cable depend. |  |
| c) | State the various parts of a cable. Also state the function of each part in brief. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Estimate the energy bill on a weekly basis for a house having following loads in operation, with their time duration of operation as specified below: - Fluorescent tubes - 36W, 6 Nos., 6 hours/day - Fans – 60W, 4 Nos., 9 hours/day - Electric heater - 1KW , 1 No., 2 hours/day - Geyser - 2KW, 1 No., 2 hours/day - Mixer - 750W, 2 Nos., 1 hour/day The cost of unit consumed is Rs.5/-. |  |
| b) | In which of the supply polarity is switch and fuse located? Which of the test is to be performed for such verification before a wiring installation is energised? Draw a neat circuit diagram to perform such test and list out the various steps involved. |  |
| c) | i) Give two points of distinction between the two types of conduct wiring system. ii) State any six factors affecting the selection of a type of wiring system. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Why is earthing necessary? What is earthing? |  |
| b) | State the types of earthing electrodes. What should be the nominal value of earth resistance for effective earthing? State the methods used for reducing earth resistance value. |  |
| c) | State the main parts of a transformer and give their functions. | ……2/- |
|  | -2- |  |
| d) | In case of a single phase 2.5KVA 230/115V transformer having 112 turns on primary side, determine its i) Number of turns on secondary side ii) Turns ratio iii) Voltage ratio iv) Current ratio. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the working principle of a dc motor with the help of neat diagram. By which rule we can obtain the direction of rotation of such motor. |  |
| b) | State the expression for speed in a dc motor. Also indicate each term used in the expression. |  |
| c) | i) Draw the mechanical characteristics for following: - DC shunt motor 2) dc series motor 3) dc cumulative compound motor. ii) How do you reverse the direction of rotation of dc motor? |  |
| d) | Classify 3 phase Induction motor. Also compare between them with reference to their starting torque and speed of operation. Give at lease two applications of each. |  |
| e) | i) How can you make single phase induction motor self starting? State the methods used. ii) How can direction of rotation of 1 split phase Induction motor be reversed? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Can a dc series motor be safely operated on ac supply? What modifications are to be incorporated in its construction to convert a dc series motor into ac series motor? |  |
| b) | What is the function of limit switch? Where is it used? What is the function of reversing switch? |  |
| c) | State the functions of time delay relay. Also state its types. What is its application? |  |
| d) | State the applications of i) MCCB ii) ELCB iii) HRC Fuse iv) Contactor. |  |
| e) | i) State the types of starters used in starting of following motor: 1) dc shunt motor 2) dc cumulative compound motor 3) Squirrel cage Induction motor 4) Slip ring Induction motor. ii) Also give two applications of 1) Shaded pole type 1 Induction motor 2) Universal motor. |  |

***********