## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./EC/EI

Subject: 8051 Microcontroller(EN501) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Refer to table A for SFR Addresses, whenever required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How can a programmer make register bank as the active bank, using instructions? |  |
| b) | List any three arithmetic instructions in 8051? |  |
| c) | What is the function of GATE bit in TMOD register? |  |
| d) | What is the difference between RET instruction and RETI instruction? When is RETI instruction used? |  |
| e) | Explain the instructions used to access ROM in 8051. |  |
| f) | State why the PC register is required. |  |
| g) | Explain the meaning of XCHD A,25H instruction with a suitable example. |  |
| h) | What is the function of RI flag? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on stack and stack pointer. With suitable instructions, shown how the stack is used for temporary data storage. |  |
| b) | Explain measuring of the different bits of the program status word. |  |
| c) | Write a short note on internal memory of 8051. Differentiate between Harvard architecture and Von Neumann architecture. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the differences between 8051, 8031 and 8751. |  |
| b) | Describe the function of various bits of SCON register. |  |
| c) | State the functions of following signals in 8051: (i) ALE (ii) RST(iii) WR. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the functions of various bits of TMOD register. |  |
| b) | Write a program to generate a continuous square waveform of 1 KHz frequency on pin P1.4 of 8051 using timer 0. Assume crystal frequency is 12 MHz. |  |
| c) | Explain in detail how the 8051 handle level –triggered and edge-triggered external interrupts. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Twenty bytes are stored in internal RAM locations starting from 50H. Write a program to copy these bytes to another block of internal RAM starting from 30H. |  |
| b) | Multiply the upper nibbles of two bytes stored in internal RAM locations 36H and 37H and store the 8-bit result in internal RAM location 38H. |  |
| c) | Explain the meaning, functional type and addressing mode of the following instructions: (i) DEC 45H (ii) XRL A, R5 (iii) MOV @ RO, #46H | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a program to continuously flash an LED connected pin P2.6 of 8051. The LED should be turned ON for 1ms and should be turned OFF for 1ms. Do not use the 8051 timers and assume XTAL frequency is 1MHz. |  |
| b) | Write a program to subtract the contents of external RAM location 45H from external RAM location 46H and store the 8-bit result in internal RAM location 30H. |  |
| c) | Count how many add bytes are present in a data block of size12, in internal RAM locations starting from 50H. Store the result in register B. TABLE A SFR NAME; ADDRESS. A; E0H. B; F0H. DPH; 83H. DPL; 82H. IE; A8H. IP; B8H. P0; 80H. P1; 90H. P2; A0H. P3; B0H. PCON; 87H. PSW; D0H. SCON; 98H. SBUF; 99H. SP; 81H. TMOD; 89H. TCON; 88H. TL0; 8AH. TH0; 8CH. TL1; 8BH. TH1; 8DH. |  |

***********