## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Hydraulic Machinery(ME404) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define viscosity of liquid. Differentiate between kinematic and dynamic viscosity. |  |
| b) | A certain liquid has a mass density of 1200 Kg/m3. Calculate its specific weight and specific gravity. |  |
| c) | State Pascal’s law and give two applications where this principle is applied. |  |
| d) | Differentiate between laminar and turbulent flow. |  |
| e) | State the major and minor energy losses in pipe. |  |
| f) | What do you mean by specific speed of turbine? |  |
| g) | How are reciprocating pumps classified? Give its one application. |  |
| h) | State the applications of rotary screw pump. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of the Bourdon pressure gauge and explain how it is used in the measurement of pressure. |  |
| b) | Fig.1 shows an inverted V-tube manometer connecting two pipes carrying water. Find the difference of pressure in the two pipes in KPa. Necessary readings are given in the figure. |  |
| c) | An isosceles triangular plate of 1.5m base and 3m altitude is immersed vertically in water in such a way that its vertex lies below the free surface of water and base is parallel to and at a depth of 1m from the free surface. Find the total pressure and the position of centre of pressure. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A circular plate of 2m diameter lies in oil in such a way that its plane makes an angle of 45 to the free liquid surface. The specific gravity of the oil is 0.75. Find the total pressure and position of centre of pressure. The centre of the plate lies at a depth of 1.5m from the free liquid surface. |  |
| b) | A horizontal Venturimeter with inlet diameter 150mm and throat diameter 75mm is used to measure the flow of oil of specific gravity 0.8. The reading of the differential manometer connected to the inlet is 160mm of mercury. Calculate the rate of flow if coefficient of meter is 0.98. |  |
| c) | What is a Pitot tube? Explain how is it used in the measurement of velocity of flow. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A horizontal pipe 140mm in diameter suddenly enlarges to 200mm diameter pipe. Find the loss of head due to sudden enlargement if the water is flowing at a rate of 0.05m3/s. If the direction of flow is reversed, what would be the loss due to sudden contraction? |  |
| b) | With the help of a neat sketch explain the construction and working of a Kaplan turbine. |  |
| c) | Describe the main and operating characteristics of hydraulic turbine. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the main components of a Centrifugal pump. |  |
| b) | Find the power required to drive a centrifugal pump for delivering 50 lit/s of water to a height of 20m from sump to delivery tank. Take overall efficiency as 70% and additional head of 10m to account for all types of losses. |  |
| c) | A single acting reciprocating pump running at 60 r.p.m. delivers 0.0165 m3/s of water. The diameter of the piston is 250mm and stroke length of 350mm. The suction and delivery heads are 5m and 20m respectively. Find the theoretical discharge, coefficient of discharge and the power required to drive the pump. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the operational difficulties commonly experienced in centrifugal pump and their remedies. |  |
| b) | With the help of a neat sketch explain the construction and working of a rotary gear pump. |  |
| c) | A compound piping system consisting of 450mm diameter for 900m, 375mm diameter for 450m and 300mm diameter for 300m of length is required to be replaced by a pipe of uniform diameter of 300mm. Find the equivalent length of the pipe of the same material. |  |