## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg.

Subject: Soil Mechanics(CE505) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the term void ratio. Establish the relation between void ratio and porosity. |  |
| b) | What is relative density? State the engineering significance. |  |
| c) | Explain Darcy’s law and state the validity. |  |
| d) | Define the following: i) Consolidation settlement ii) Creep of soil. |  |
| e) | Define the term bearing capacity and mention the bearing capacity values of any three types of soil. |  |
| f) | Define the term area ratio and recovery ratio of sample and state the significance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch the phase diagram for a soil and indicate the volumes and weights of the phases on it. |  |
| b) | Determine the i) Water content ii) Dry density iii) Bulk density form the following data: i) Size of sample 4cm diameter and 9cm height ii) Wet weight=2.50N iii) Dry weight= 1.50N iv) Specific gravity=2.65. |  |
| c) | Define and explain: Liquid limit, Plastic limit and Shrinkage limit. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain BLS soil classification system. |  |
| b) | Draw a neat sketch of particle size distribution curve and explain well graded, uniformly graded and poorly graded soil. |  |
| c) | Enlist and explain the factors controlling permeability of soil. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the coefficient of permeability from the following data: i) Length of sand sample=28cm ii) Area of cross-section=30cm2 iii) Head of water=40cm iv) Discharge=200ml in 110 seconds. |  |
| b) | Explain the principles of direct shear test. What are the advantages and disadvantages? |  |
| c) | Enlist and explain the factors contributing shear strength of soil. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the term compaction and explain standard compaction test. |  |
| b) | Enlist the objects of soil exploration. Also mention how location, depth and spacing of exploration is decided. |  |
| c) | Explain the factors controlling compaction. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Soil improvement with Soil Reinforcement Technique |  |
| b) | Types of shear failure in soils |  |
| c) | Settlement and types of settlements |  |
| d) | Presentation of soil investigation results |  |
| e) | Total stress and effective stress |  |
| f) | Plate load test |  |
| g) | Importance and objects of ground improvement |  |

***********