## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Welding Technology-I(4237)/(FE301) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the advantages of welding as a Fabrication process over other joining processes. |  |
| b) | What is molten pool or weld puddle or molten puddle? |  |
| c) | Why is manifolding system for gas welding required? |  |
| d) | Explain the different types of free flight metal transfer in Arc welding. |  |
| e) | Differentiate between welding and brazing as joining process. Mention any two advantages of brazing over welding. |  |
| f) | State the applications of sprayed coatings. |  |
| g) | Explain why Aluminium and its alloys are more difficult to weld than structural steel even though its melting point is lower. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the factors affecting arc blow, types of arc blow and remedies for arc blow. |  |
| b) | Explain with a neat sketch the working of a two stage gas regulator. |  |
| c) | Explain the procedure to set, light, adjust, use and extinguish the oxy-acetylene flame welding equipment. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch drooping power source characteristics. |  |
| b) | Differentiate between constant current and constant voltage power source characteristics. |  |
| c) | What are the main precautions to be taken while carrying out metal arc welding and oxy-acetylene gas welding? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief the principles of good soldering process. |  |
| b) | Explain briefly the common welding defects and their rectification. |  |
| c) | Explain the procedure of vacuum brazing. Differentiate between vacuum brazing and furnace brazing. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe in brief, why edge preparation of base metal is essential. |  |
| b) | Explain the formula to determine ‘Carbon Equivalent’ of alloy steel. Explain its effects on their weldability. |  |
| c) | Explain with a neat sketch construction of a cutting torch and state its working principle. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Wire spraying process |  |
| b) | Oxy-natural gas welding |  |
| c) | Silver brazing |  |
| d) | Protection of welders |  |
| e) | Low Hydrogen Electrodes |  |

***********