## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Machine Tools & Production Process-II(4060) [old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is trepanning? |  |
| b) | How is horizontal boring machine specified? |  |
| c) | What do you understand by length of stroke and position of stroke in regard to shaping machine? |  |
| d) | Differentiate between puncher slotter and precision slotter. |  |
| e) | Give classification of planning machine. State one function of each type. |  |
| f) | Draw a neat labelled sketch of a milling machine arbor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate the machining time to complete a cut on a milling machine, given the following data: Length of table travel=260mm; Feed per tooth =0.1mm; RPM of cutter =450rpm; Number of teeth on cutter =40. |  |
| b) | Explain the difference between up milling and Down milling with neat sketches. |  |
| c) | Compare a plain milling machine with a universal milling machine giving the advantages and disadvantages of each and their uses. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by quick return mechanism of a shaper? Describe any one in detail. |  |
| b) | What are the various operations carried out on a shaper? Describe any two of them in detail with neat sketches. |  |
| c) | With a sketch explain the main parts of a standard shaper. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various methods of holding tools in a drilling machine spindle? Explain any two in detail with neat sketches. |  |
| b) | A drill having 20mm  diameter is used for drilling a hole in a mild steel block. The total length of travel of drill is 400mm, feed per revolution is 0.5mm and has cutting speed of 25mm/min. Find the machining time in minutes. |  |
| c) | Explain the process of gear cutting on a milling machine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is jig boring machine? State the various methods of locating holes in jig boring. Explain any one in detail. |  |
| b) | With a neat block diagram explain the main parts of a planer. |  |
| c) | Explain how two opposite sides of a rectangular block will be machined on a slotting machine. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Radial Drilling Machine |  |
| b) | Horizontal boring machine |  |
| c) | Operations of planer |  |
| d) | Safety precautions while working on slotter |  |
| e) | Work holding devices used on milling machine |  |

***********