## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT

Subject: Transportation Engineering-I(CE304) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is importance of transportation in National Development? |  |
| b) | Define the following terms: i) Formation width ii) Shoulders iii) Balancing of earthwork |  |
| c) | Explain briefly the terms: i) Prime coat ii) Tack coat iii) Seal Coat. |  |
| d) | State circumstances for construction of Asphalt roads over Concrete roads. |  |
| e) | What is object and scope of traffic engineering? |  |
| f) | Draw a neat sketch of balanced cantilever bridge. |  |
| g) | Draw a neat labelled sketch showing any two types of culverts. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail classification of roads. |  |
| b) | Write the definition, types with sketches & IRC specification for any two of the following: i) Camber ii) Sight distance iii) Gradient. |  |
| c) | What is soil stabilization? Explain necessity and method of soil stabilization. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure of construction of WBM road and give precautions to be taken in rolling. |  |
| b) | State the general defects noticed in bituminous roads during monsoon. Explain the appropriate remedial measures in each case. |  |
| c) | With a neat sketch explain continuous bay and alternate bay methods of construction of concrete roads. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention common defects in concrete roads and explain its maintenance procedure. |  |
| b) | Draw a neat cross-section for hill road and explain its maintenance. |  |
| c) | Explain various methods of providing the surface drainage in case of urban roads. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the component parts of a bridge with a neat sketch. |  |
| b) | Explain following types of bridges with a neat sketch: i) Pre-stressed concrete bridge ii) Suspension bridge. |  |
| c) | Explain incremental push launching method of construction of bridges. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Widening of Roads on curves |  |
| b) | Geometric design of hill roads |  |
| c) | Road Arboriculture |  |
| d) | Bridge bearings |  |
| e) | Flyover |  |

***********