## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Marine Engineering-III(4342) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any 5 from the remaining questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Mooring winches |  |
| b) | Types of Non-return valves |  |
| c) | Fittings on a steam pipe on deck |  |
| d) | Difference between air conditioning and refrigeration system |  |
| e) | Emergency source of power on a ship |  |
| f) | Methods of securing pipe lines on a ship |  |
| g) | Marpol requirements for disposal of sewage |  |
| h) | Fire hoses and fire hydrant. |  |
| Q.No.2. | Sketch & describe an Anchor windlass. Explain its working. Show how it is also used as a mooring winch. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | What is a bow thruster? What is its purpose? Where is it fitted and how it is driven? | (6) |
| b) | Briefly describe a stabiliser for a ship. Explain its purpose. | (6) |
| Q.No.4. | Sketch & describe a fuel oil service system of a diesel engine, from service tank to the engine. Name all the equipments fitted in the system. | (12) |
| Q.No.5. | Sketch & describe a vapour compression refrigeration system as used on a ship. How are different temperatures maintained in meat room, fish room and vegetable room? | (12) |
| Q.No.6. | Explain with the aid of sketches, the basic principle of working of an electric motor. How is the power produced in an alternator? | (12) |
| Q.No.7. | Sketch a power distribution system of a ship. How is shore connection connected to main bus bars? What safeties are provided in shore connection? | (12) |
| Q.No.8. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Marpol regulation for disposal of garbage |  |
| b) | International shore coupling |  |
| c) | Refrigerants used in a refrigeration system |  |

***********