## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./EC

Subject: Advanced Microcontrollers(EN613) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State 5 Interrupts of 8051 along with its ISR addresses. |  |
| b) | Write only one instruction to enable timer 0 Interrupt, Serial Interrupt & external Hardware Interrupt. Is there any other way to enable above Interrupts? |  |
| c) | Explain in brief about Serial Interrupt of 8051. |  |
| d) | Write an instruction to use SCON register for transmission & reception of data in mode 1. Which register is used to double the baud rate? |  |
| e) | Which timer of 8051 is used to set the baud rate? In which mode it is used? |  |
| f) | State different software development tools. |  |
| g) | Draw Interfacing diagram of one 7 segment display to 8051 microcontroller. Assume ports for Input & output. |  |
| h) | State features of PIC microcontroller. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write an assembly language program to generate a square wave of 10KHZ frequency on p1.2 bit using timer 0 Interrupt. Assume Xtal frequency = 11.0592 MHz. |  |
| b) | List the steps to use timer 1 Interrupt of 8051 to generate a square wave of certain frequency on any port of microcontroller. Use timer in mode 2. |  |
| c) | Two switches are connected to pins p3.2 & p3.3 of 8051 microcontroller. When switch is pressed the corresponding line goes low. Write an assembly Language program to - Light all the LEDS connected to port 0 for short duration, if the first switch is pressed. - Light all the LEDS connected to port 2 for short duration, if the second switch is pressed. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For the Xtal frequency of 11.0592 MHz, find TH1 value for baud rates: i) 9600 ii) 2400 iii) 1200. If SMOD is mode 1, what will happen with above baud rates? |  |
| b) | Write the steps to receive the data serially in 8051 system. |  |
| c) | Write an assembly language program to read port 0 & port 1 of 8051 one after the other and transmit that data serially, continuously. Xtal frequency = 11.0592 MHz, Baud rate= 4800. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different serial protocols? Explain in brief. |  |
| b) | Write short notes on: i) IDE ii) Compiler iii) Simulator iv) Debugger. |  |
| c) | Explain in brief ICE, Target board & Device programmer. |  |
|  |  | …….2/- |
|  | -2- |  |
| d) | Draw Interfacing diagram of 4x4 matrix keyboard to P1 & P2 of 8051 as Output & Input respectively. Write an assembly Language program to detect & identify key activation and display it on P0. (ASCII code of key should be displayed). |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw an Interfacing diagram of stepper motor to 8051 port 1. Write an assembly language program to rotate it clockwise direction continuously in full step sequence. |  |
| b) | Draw an Interfacing diagram of ADC (0808) to 8051. Write an assembly Language program to start the conversion & store the result in Accumulator. Xtal frequency = 11.0592MHZ. |  |
| c) | Draw Interfacing diagram of DAC (0808) to port 1 of 8051. Write an assembly Language program to generate 5 steps staircase waveform. Assume delay of 100msec for width of step. Xtal frequency = 11.0592 MHZ. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw only, the Interfacing diagram of LCD to 8051. |  |
| b) | Draw functional block diagram of PIC 18FXX & explain in brief. |  |
| c) | Explain in brief registers of PIC 18FXX microcontroller. |  |
| d) | Compare all serial protocols. |  |
| e) | Explain in brief memory organization of PIC 18FXX microcontroller. |  |

***********