## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-II(4041) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric charts, Steam Tables and Mollier’s

chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by ‘Critical pressure ratio’ in the nozzle? Explain its significance. |  |
| b) | Define with reference to steam turbines: i) Diagram factor ii) Stage efficiency. |  |
| c) | State the functions of a condenser in steam power plant. |  |
| d) | State three major differences between S.I engines and C.I. engines. |  |
| e) | What is turbocharging in I.C. engines? What are its advantages? How is it different from supercharging? |  |
| f) | State any six desirable properties of a refrigerant. |  |
| g) | State the advantages of gas turbine over steam turbine. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the battery ignition system used in multicylinder petrol engine. |  |
| b) | With the help of a neat sketch, explain the construction and working of any one type of surface condenser. |  |
| c) | What is governing of steam turbines? With the help of a neat sketch, explain the nozzle control governing of steam turbines. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rope brake dynamometer has brake wheel diameter of 600mm and the diameter of rope is 5mm. The dead load on the brake is 210N and spring balance reads 30N. If the engine makes 450 r.p.m, find the brake power developed. |  |
| b) | In a De-Lavel turbine, the steam enters the wheel through a nozzle with a velocity of 500 m/s and at an angle of 20 to the direction of motion of the blade. The blade speed is 200 m/s and the exit angle of the moving blade is 25. Find the inlet angle of the moving blade, exit velocity of steam and its direction and work done per kg of steam. |  |
| c) | Calculate the vacuum efficiency from the following data: Vacuum at steam inlet to condenser = 700mm of Hg; Barometer reading = 760 mm of Hg; Hot well temperature = 30C. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the pressure feed lubrication system in internal combustion engines. |  |
| b) | Explain with neat sketches, the working principle of 2-stroke cycle petrol engine. |  |
| c) | A petrol engine working on the Otto cycle has a cylinder of diameter 200mm and stroke 250mm. The clearance volume is 1570cc. Find the air standard efficiency. Assume Cp=1.004 kJ/KgK and Cv = 0.717 KJ/KgK fo air. | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of an ice plant. |  |
| b) | With the help of T-S and P-h diagrams, explain the vapour compression refrigeration cycle with superheating and sub cooling. |  |
| c) | With the help of a neat sketch, explain the working of a closed cycle gas turbine. State its advantages over open cycle gas turbines. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Briefly explain the regenerative and reheat cycles in steam turbines. |  |
| b) | Draw a neat sketch of induced draft cooling tower and briefly explain its working. |  |
| c) | Write a short note on application of gas turbines. |  |
| d) | Dry saturated steam at 5 bar with negligible velocity, expands isentropically in a convergent nozzle to 1 bar and dryness fraction 0.94. Determine the velocity of steam leaving the nozzle. |  |

***********