## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A jet of water 30mm in diameter having a velocity of 35 m/s strikes normally a flate smooth plate. Determine the thrust on the plate if the plate is moving in the same direction as the jet with a velocity of 8 m/s. |  |
| b) | Name any three types of impellers used in centrifugal pumps and state their use. |  |
| c) | What is the significance of specific speed of water turbines? |  |
| d) | What are the functions of air vessel? |  |
| e) | What is the function of quick exhaust valve? Write its symbol. |  |
| f) | What is the function of oil seal? State their types. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Pelton wheel is required to generate 3750 KW under an effective head of 400m. Find the total flow, and size of the jet. Assume generator efficiency of 95%, overall efficiency of 80%, coefficient of velocity 0.97, speed ratio 0.46. |  |
| b) | Draw a schematic diagram of a Francis turbine and explain briefly its construction and working. |  |
| c) | What do you mean by Net Positive Suction Head (NPSH)? What are the effects of cavitation? List the factors which facilitate onset of cavitation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Kaplan turbine produces 44000 KW under a heads of 24.7 m with an overall efficiency of 90%. Taking speed ratio as 1.6, flow ratio as 0.5 and hub diameter as 0.35 times the outside diameter, find the runner diameter and speed of the turbine. |  |
| b) | What is the significance of characteristics curves for centrifugal pumps? Draw the (i) Operating characteristics curves (ii) Muschel curves (constant efficiency). |  |
| c) | A double acting reciprocating pump runs at 90 rpm. The diameter and stroke are 100 mm and 250 mm respectively. The suction pipe is of 100 mm diameter and 5m long. Calculate the maximum permissible suction lift assuming no air vessel if fitted and separation occurs at 2m of water absolute. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A centrifugal pump delivers 0.03m3/s of water to a height of 18m through a pipe 90m long and of 100mm diameter. If the overall efficiency of the pump is 75%, find the power required to drive the pump. Take f=0.012 |  |
| b) | With the help of neat sketch explain the governing mechanism of an impulse turbine. | …….2/- |
|  | -2- |  |
| c) | Define indicator diagram. Draw an indicator diagram, considering the effect of acceleration and friction in suction and delivery pipes. Write the mathematical expression for work done per second in case of a double acting reciprocating pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working of a reciprocating pump. |  |
| b) | Explain the construction and working of a screw pump with the help of a neat sketch. |  |
| c) | Describe any Vacuum pump with help of a neat sketch. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With help of a neat sketch explain the construction and working of Non-return valve. |  |
| b) | What are the different types of direction control valves used in pneumatic circuits and how they are actuated? |  |
| c) | Explain meter-in type of hydraulic circuit with the help of a neat sketch. |  |

***********