## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT

Subject: Public Health Engineering(4015)/(CE404) [Old/Rev.]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write any 6 points to justify why our environment should be protected. |  |
| b) | Give any three comparisons on ground water sources and surface water sources. |  |
| c) | Specify the units, write the definition and give the safe permissible limits of the following for potable water: (i) Turbidity (ii) Colour (iii) Chlorides. |  |
| d) | What are the various losses & wastes of water in Indian city water supply? Specify the value of this loss in estimating the total demand of water for an Indian city. |  |
| e) | Draw a neat flow diagram of a typical water treatment plant. |  |
| f) | Draw a neat sketch of overhead water tank for a house and show all the valves, inlets, outlets, etc. |  |
| g) | Define: sewage, sullage, outfall sewer, invert level of sewer, self cleansing velocity, dry weather flow. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | (i) What are the waterborne diseases? Name any three. (ii) List out any 6 precautions to be taken to avoid spread of water borne diseases in a city or town. |  |
| b) | Explain what is the importance of study of variations in demand. What are the various variations in demand and specify their values in percentage of average demand. |  |
| c) | What is the principle of sedimentation? What are the factors affecting sedimentation? What is meant by surface loading? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled sketch of a rapid sand filter. Explain the various components such as base material, filter media, back washing facilities, inlets & outlets etc. Also, explain its working and backwashing with reference to the various valves numbered & shown in the sketch. |  |
| b) | Explain with neat sketches the four types of layout of distribution system of water for a town. Mention also suitability of each type of layout. |  |
| c) | (i) Mention the various types of pipes used in water supply. Indicate suitability of each. (ii) Explain with sketch, a suitable joint in concrete pipes and a joint in steel pipes. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the three systems of water supply plumbing in a building based on pressure available and supply type? Explain each with a neat sketch staring from public water line to the taps, in a two storeyed building. Label all the parts such as ferrul joint, meter, pump,etc. | ……2/- |
|  | -2- |  |
| b) | (i) What are the manholes and lamp holes? Where are they to be located? (ii) Explain with a neat sketch, a drop manhole. |  |
| c) | (i)Enlist the treatment units in primary, secondary treatment & tertiary treatment of sewage in a sewage treatment plant. (ii)Distinguish between aerobic and anaerobic decomposition. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is biological treatment of sewage? What are the types? Explain any one biological treatment unit with necessary sketch & its working. |  |
| b) | What are single stack, one pipe and two pipe systems in a house plumbing? Explain with sketch the two pipe system of sanitary plumbing for a 3 storeyed building. |  |
| c) | (i) What are the traps in building plumbing and what are their functions? Show neat sketches of any three types of traps and indicate clearly the effective water seal in each. (ii) What are the inspection chambers? Draw a neat cross section of an inspection chamber and mention where they are required. |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | To keep the town with good environment and to avoid spared of any disease, what precaution/action should be followed by each citizen? Also include the points on duties of a Public Health Engineer to achieve the same. |  |
| b) | Write any four points on factors to be considered while selecting a source of water for the supply to a town. |  |
| c) | Write short notes on sewer ventilation with sketches & mention the necessity of sewer ventilation. |  |
| d) | Define: dry refuse, garbage, rubbish. Specify some constituents of each. |  |
| e) | How do you segregate the solid wastes from domestic/commercial buildings so that their disposal/ recycling becomes easier? Specify the constituents of each group segregated. |  |
| f) | Explain compost making method of disposal of domestic waste in villages. |  |

***********