## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Thermal Engineering(4052) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is boiler draught? |  |
| b) | Explain what are the losses occurring in steam turbine. |  |
| c) | Define IP, BP and FP. |  |
| d) | How are reciprocating air compressors classified? Explain the need of clearance volume in a reciprocating compressor. |  |
| e) | Define thermal conductivity of material. |  |
| f) | Explain heat transfer by convection. |  |
| g) | What is the function of a cooling tower? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a Babcock and Wilcox boiler and label any five parts. |  |
| b) | What are the differentiating features between a water tube and fire tube boiler? |  |
| c) | Explain construction of an Air pre-heater with a neat sketch and explain the function of it. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the dryness fraction, enthalpy, entropy and internal energy of 1.5kg of steam when its pressure is 2 MPa and specific volume is 0.09 m3/kg. |  |
| b) | Draw a neat sketch of separating and throttling calorimeter and explain its use in determination of quality of steam. |  |
| c) | State the various methods of governing of steam turbine. Explain any one with a sketch |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of the Otto cycle with the help of theoretical indicator (P-V) diagram. |  |
| b) | Explain with the help of a suitable sketch, the working of four stroke diesel engine. |  |
| c) | Mention any six advantages of multi-stage reciprocating air compressor over that of a single stage type. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The glass windows of a room have a total area of 10m2 while their thickness is 4mm. Find the rate at which heat escapes from the room by conduction when the temperature difference is 20C. Take ‘K’ for glass as 0.744 w/mk. |  |
| b) | With the help of a neat sketch, explain the working of a parallel flow jet condenser. |  |
| c) | Calculate vacuum efficiency from the following data: Vacuum at steam inlet to condenser 700mm Hg; Barometer reading 760mm Hg; Hot well temperature 30C |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is the unit of refrigeration? Define it. |  |
| b) | What refrigerant is used in automobiles for air-conditioning and give three properties of it? |  |
| c) | Explain under cooling with the help of P-V diagram. |  |
| d) | Name any three refrigerants, and explain any one in brief. |  |

***********