## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Food Technology

Subject: Food Engineering-I(4265)/(FT304) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | What is thermal resistance? Give one example of an insulator and good conductor of heat. |  |
| b) | Give an expression for overall heat transfer co-efficient. |  |
| c) | Explain free and forced convection. |  |
| d) | What is votator? Give its one application. |  |
| e) | Define wet-bulb temperature. |  |
| f) | List the types of air–flow in a drier. |  |
| g) | Give a neat sketch of Forced circulation evaporator. |  |
| h) | State the advantages of vacuum concentration. |  |
| i) | Give the unit of Refrigeration. Name two refrigerants used in food industry. |  |
| j) | Name three food products frozen using a Plate freezer. |  |
| k) | What is Steam distillation? Give one example. |  |
| l) | State the advantages of Freeze-drying. |  |
| Q.No.2. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Derive an expression for conduction of heat across homogeneous wall. |  |
| b) | Explain the working of extended surface heat exchangers. |  |
| c) | For a boiling liquid in a pipe externally heated, what is its critical temperature difference? |  |
| d) | List out the advantages of Plate heat exchangers and state the applications. |  |
| e) | With a neat diagram give the expression for LMTD (log-mean temperature difference) for co-current and counter current flow. |  |
| Q.No.3. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | 1000Kg of Mango Pulp of 12.1 solids is concentrated to 45% solids. How much Mango concentrate is obtained? |  |
| b) | Show Falling rate period during drying with the help of a graph. List the factors which affect the Falling rate period. |  |
| c) | Differentiate between free moisture and bound moisture. |  |
| d) | With a neat diagram explain the working of steam jet ejector. |  |
| e) | Suggest a drier for the following: - Instant soup mix ii) Onion flakes iii) Apple rings iv) Tea-leaves v) Milk vi)Orange juice |  |
| Q.No.4. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain the working of climbing film evaporator. State its uses. |  |
| b) | What is Multiple effect evaporator? List the different methods of feeding a multiple effect evaporator with the help of diagram. |  |
| c) | Explain the working of Sling Psychrometer to measure humidity. Define Relative humidity. |  |
| d) | Explain the working of Cooling Tower. What do you mean by ‘Saturated air’? |  |
| e) | Explain the working of cabinet drier. List the disadvantages. | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the working of simple vapour compression refrigeration system. |  |
| b) | How to calculate the refrigeration load to store a product from its initial temperature to the desired temperature? |  |
| c) | Explain the working of Plate freezer. State the disadvantage of Air blast freezing. |  |
| d) | Explain a simple distillation process with an example. What is Fractionation? |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Spray drier and its parts |  |
| b) | Freeze-drying |  |
| c) | Principle of absorption refrigeration |  |
| d) | Fluidised bed freezer |  |

***********