## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-II(AT602) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by supercharging? Give three objectives of supercharging. |  |
| b) | Explain how can engine design change result in control of emissions. |  |
| c) | Describe the principle of operation of a Petrol EFI system? |  |
| d) | List out the advantages of CRDi system over a conventional fuel injection system. |  |
| e) | What do you mean by Refrigeration Evacuation, in HVAC system. |  |
| f) | List out 4 advantages of Scan Tools. |  |
| g) | What are diagnostic trouble codes? How many characters are displayed and what is the significance of each character? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out any three types of Superchargers. Draw a neat sketch and explain any one in detail. |  |
| b) | Explain super charging of SI engines. What are the effects on the performance of an automobile? |  |
| c) | What are exhaust gas analyzers? Explain Concentration Sampling process. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the ECM controlled EGR system. |  |
| b) | Describe a 3-way catalytic converter using a neat diagram. |  |
| c) | List the types of petrol EFI systems used. Also explain an MPFI system with a neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any two of the following sensing devices: i) O2 Sensors ii) Thermistors iii) Potentiometers |  |
| b) | List out five main components of a CRDi system. Explain any two in brief with sketches. |  |
| c) | Draw a neat sketch of an HVAC system. List out all the components and explain the working of an Accumulator. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the Vane type AC compressor. |  |
| b) | Gives the purpose, limitations and advantages of Non-Powered test lights. |  |
| c) | What do you understand by Pin-point testing? Explain testing of PCM. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write a short note on electronically controlled Wastegate. |  |
| b) | Explain a P.C.V system. |  |
| c) | Describe in detail Throttle body assembly. |  |
| d) | Write a short note on Technical Service Bulletins. |  |
| e) | Describe the purpose and function of Breakout Boxes |  |
| f) | Explain Refrigerant Recovery Process for an HVAC system. |  |

***********