## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mechanical Engg./A/SB

Subject: Mechanical Engineering Materials(4031) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following terms: i) Durability ii) Elasticity iii) Toughness iv) Hardness. |  |
| b) | Differentiate between thermosetting and thermoplastic. |  |
| c) | Give the composition, properties and application of Muntz metal. |  |
| d) | What do you understand by Non-destructive testing? |  |
| e) | Give the classification of steels based on heat content. |  |
| f) | Differentiate between Normalising and Annealing. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is ‘Free cutting steel’? State its properties and applications. |  |
| b) | Explain the effect of following elements on structure and properties of cast-iron: i) Silicon ii) Manganese. |  |
| c) | Explain the following: i) Bearing metals ii) Metals for Nuclear Energy. |  |
| d) | State properties and uses of Grey Cast Iron, Malleable Cast Iron, Chilled Cast Iron and Alloy Cast Iron. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw an iron-carbon phase equilibrium diagram showing all critical points and critical range. |  |
| b) | Explain the TTT diagram with neat sketch. |  |
| c) | Explain ‘flame hardening’ and ‘induction hardening’. State application of each process. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the principle of magnetic particle test. What are its advantages and limitations? |  |
| b) | Compare X-ray and  ray radiography methods. |  |
| c) | What is rubber? How is it different from plastics? Name the various types of rubber. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss various properties of refractories in brief. |  |
| b) | Explain the following heat insulating materials: i) Asbestos ii) Glass wool iii) Cork. |  |
| c) | With reference to plastics explain the following: i) Phenolics ii) Polyethylene iii) Nylon. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is borosilicate glass? State its properties & uses. |  |
| b) | Write short note on Ultrasonic detector test. |  |
| c) | Write short note on types of rubber: i) Neoprenes ii) Nitryl Rubber iii) Silicon Rubber |  |

***********