## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(4104)/(EE301) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Find voltage drop across R1 & R3 of circuit shown in fig.1, Q.No.1a by using KCL & KVL. |  |
| b) | For the circuit shown in fig.2, Q.No.1b, find the value of equivalent resistance across terminals A & B. |  |
| c) | For the figure shown in fig.3, Q.No.1c, find the - p.f. of the circuit & circuit elements if Vm=102V & Im=52A - state whether circuit is inductive or capacitive. |  |
| d) | With the help of a power triangle, explain the various types of power in an a.c. R-L circuit. |  |
| e) | Explain how an unsymmetrical system of 3-phase currents can be resolved into 3 symmetrical component systems. |  |
| f) | Draw the circuit diagram of Desauty’s bridge. Give its equation at balance. List down atleast one advantage and one disadvantage of it. |  |
| g) | Draw circuit diagram of Maxwell’s inductance bridge & give its equation at balance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the thevenin’s equivalent circuit across terminals a & b for the circuit shown in fig 4, Q.No.2a. |  |
| b) | Compute the power dissipated in the 9Ω resistor of fig.5, Q.No.2b by applying the superposition theorem. All resistances are in ohms & voltage and current sources should be treated as ideal sources. |  |
| c) | Find the branch currents in the circuit shown in fig.6, Q.No.2c by using Nodal analysis. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A resistance R, inductance of 0.01H and capacitance of C are connected in series. When a voltage of V = 400 cos300t volts is applied to the series combination, the current flowing is 102 cos (300t+40º)Amps. Calculate R and C. | ……2/- |
|  | -2- |  |
| b) | Two impedances consists of i) resistance of 15 ohms & series connected inductance of 0.04H and ii) resistance of 10 ohms, inductance of 0.1H and a capacitance of 100µF all in series. They are connected in series across a 230V, 50Hz supply. Find i) Current drawn ii) Voltage across each impedance iii) Individual & total power. Draw the phasor diagram. |  |
| c) | For the circuit shown in fig.7, Q.No.3c, calculate the i) applied voltage, active power, reactive power and apparent power. Construct the power triangle. What is the p.f. of the circuit? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define i) Resonance ii) Resonance frequency iii) bandwidth iv) Quality factor. Also, explain why the circuit impedance is always minimum at resonance. |  |
| b) | A series R-L-C circuit consist of R=1000Ω, L=100mH and C=10 picofarads. The applied voltage to the circuit is 100V. - Find the resonant frequency of the circuit - Find the quality factor of the circuit at resonant frequency - At what angular frequencies do the half power points occur? - Calculate the bandwidth of the circuit. |  |
| c) | Calculate the impedance of the parallel tuned circuit as shown in fig.8, Q.No.4c at a frequency of 500KHz and for bandwidth of operation equal to 20KHz. The resistance of the coil is 5 ohms. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A balance star connected load of (8 + j6)Ω per phase is connected to a balanced 3-phase, 400V supply. Find the line current, p.f., power and total volt-amperes. |  |
| b) | Three 100Ω non-inductive resistances are connected in i) Star & ii) Delta across a 400V, 50Hz, 3-phase mains. Calculate the power taken from supply in each case. In the event of one of the three resistances get open circuited, what would be the value of total power taken from the mains in each case of the two cases? |  |
| c) | Three identical impedance are connected in delta to a 3-phase 400V, 50Hz. The line current is 35A and the total power taken from supply is 15KW. Calculate the resistance and inductance of each impedance. Also calculate the total KVAR. |  |
|  |  | …..3/- |
|  | -3- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find out the positive, negative and zero phase sequence components of the following set of three unbalanced voltage vectors. VA=1030º, VB=30-60º, VC=15145º |  |
| b) | The four arms of a Hay’s a.c. bridge are arranged as follows: AB is a coil of unknown impedance, BC is a non-reactive resistance of 1000 ohms. CD is a non-reactive resistance of 833 ohms in series with a standard capacitor of 0.38µF. DA is a non-reactive resistor of 16800 ohms. If the supply frequency is 50Hz, determine the inductance and resistance of the unknown coil at balance condition. |  |
| c) | Explain what do you mean by transients. Explain the different types of transients. |  |