## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Measurements & Inststr.(4103)/(EE303)[Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define: i) True value ii) Reproducibility iii) Sensitivity. |  |
| b) | Briefly explain any one effect of electric current and state its application in electrical measurement. |  |
| c) | Explain the term ‘controlling torque’. State the significance and function of controlling system in an analog indicating instrument. |  |
| d) | List the four main parts of operating mechanism of 1 induction type of energy meter. Briefly explain any one part. |  |
| e) | Explain how speed and friction errors can be adjusted in single phase induction type energy meter. |  |
| f) | State the formula and explain the principle of measurement of low resistance using Kelvin’s double bridge. |  |
| g) | State and explain the need for use of instrument transformer in the field of measurement. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the classification and briefly explain the various types of errors encountered in measurement. How can there errors be minimized? |  |
| b) | State the need of damping torque in an analog indicating type of measuring instrument. List the methods of providing damping torque. Explain any one such method with neat diagram. |  |
| c) | With a neat diagram explain the construction and working of three phase induction type energy meter. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain the construction and working of an attraction type of Moving Iron (MI) instrument. |  |
| b) | Briefly explain the various errors associated with PMMC type of measuring instrument and state methods adopted to minimize the same. |  |
| c) | With the help of circuit diagram explain the construction and concept of rectifier type of measuring instrument using half wave rectifier circuit. State its advantages. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat circuit diagram explain the concept and method of measurement of power in 3, 3 wire system using 2 wattmeter method for a balanced star connected load. Also draw necessary phasor diagram. |  |
| b) | Write a short not on ‘Errors and their compensation’ with respect to dynamometer type of wattmeter. |  |
| c) | Explain the construction and principle of operation of Dynamometer type wattmeter. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat circuit diagram explain the voltmeter-Ammeter method for measurement of medium resistance. State its advantages and limitations. |  |
| b) | With neat diagram explain the construction of Wheatstone bridge. Also explain the concept for measurement of medium resistance. |  |
| c) | With neat diagram explain the extension of range of 3 energy meter using CT and PT. State any two advantages of this method. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the methods of extension of range of DC and AC Voltmeter and Ammeter. With neat diagram explain the method of extension of range of DC voltmeter. |  |
| b) | Write short note on ‘Earth tester and measurement of earthing resistance’. |  |
| c) | Write a short note on ‘Vibrating reed type Frequency meter’. |  |

***********