## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(4335)/(SH401) [Old/Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Bale and grain capacity |  |
| b) | Side launching |  |
| c) | Cross curves of stability |  |
| d) | Permeability and margin line |  |
| e) | Statical stability curve |  |
| f) | Wave bending moment |  |
| g) | Stowage factor |  |
| Q.No.2. | A box shaped vessel 100m long, 20m wide, 12m deep is floating in salt water on an even keel at 6m draft. A forward compartment is 10m long, 20m wide and extends from the outer bottom to a watertight flat, 4m above the keel. The compartment contains cargo of permeability 25 per cent. Find the new draft if this compartment is bilged. KG=2.5m | (12) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with sketch the concept of pivotting and tipping in stern launching of vessel. |  |
| b) | Explain the determination of GZ curves from cross curves. |  |
| c) | When inclined a ship of 8000 tonnes displacement has the following righting levers. Heel; 0; 15; 30; 45; 60. GZ (m); 0; 0.2; 0.3; 0.32; 0.24. |  |
|  | Calculate the dynamic stability to 60 heel. |  |
| Q.No.4. | From the section shown in fig.1, calculate i) The position of neutral axis ii) Section modulus at deck and keel. | (12) |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | How is the weight curve arrived at in ship strength calculations? |  |
| b) | Explain how the section modulus of midship section can be improved when needed. |  |
|  | ### OR |  |
| Q.No.5. | A box shaped barge of length 50m is of uniform construction and has displacement of 600 tonnes in lightship condition. The large has equal hold lengths with loading as shown below: Hold No.5; Hold No.4; Hold No.3; Hold No.2; Hold No.1. 400 tonnes; 300 tonnes; 400 tonnes; 500 tonnes; 300 tonnes. | (12) |
|  | Draw i) Shear force diagram ii) Determine the position of BMmax. |  |
|  |  | …..2/- |
|  | -2- |  |
|  | (Only for students of revised scheme) |  |
| Q.No.6. | The KN values of a ship are as follows: Angles; 0; 15; 30; 45; 60; 75; 90. KN; 0; 2.5; 4.94; 6.88; 8.20; 8.89; 8.94. | (12) |
|  | Plot the GZ curve for KG=9m and determine the vanishing angle of stability. |  |
|  | (Only for students of old scheme) |  |
| Q.No.6. | An inclining experiment was carried out on a ship of 8000 tonne displacement. A mass of 10 tonne was moved 14m across the deck causing a pendulum 8.5m long to deflect 110mm. The value of KM=7.15m. Calculate the height of the center of gravity above the reel. | (12) |