## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Mining Engg.

Subject: Mining Technology(MN302) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Enumerate factors affecting selection of drilling method. |  |
| b) | State essential constituents of explosive and their functions. |  |
| c) | With sketch explain pyramid cut blasting pattern. |  |
| d) | Explain roof stitching. |  |
| e) | Enumerate purposes of raise and winze. |  |
| f) | Explain mechanism of percussive drilling. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain surface arrangement for rotary drilling. ### OR |  |
| a) | Compare rotary drilling with percussive drilling. |  |
| b) | Explain Booster explosive. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Write classification of explosives based on Indian explosive rule. ### OR |  |
| a) | Explain substitutes of explosive. |  |
| b) | Explain control blasting. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Define powder factor and explain steps in powder factor calculation. OR |  |
| a) | A blast pattern is required with: Burden = 2.7m; spacing -3m; Hole diameter 89mm; Hole depth = 8m; Number of holes – 50; stemming = 2.5m; Explosive – ANFO 5.6 = 0.85 Kg/l. |  |
| b) | Explain power support. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain safari support. |  |
| b) | With sketch explain cemmentation method of shaft sinking. ### OR |  |
| b) | Explain problem associated with winzing and how it can be reduced. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Deepening and widening of shaft |  |
| b) | Raise borer |  |
| c) | Precaution against ground vibration due to blasting |  |
| d) | Storage of explosive |  |

***********