## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111)/(EE403) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain any two methods of temperature control in resistance heating. |  |
| b) | Give any three applications of Dielectric heating. |  |
| c) | Give three points of comparison between Arc welding and Resistance welding. |  |
| d) | Explain the concept of production of light in gas discharge lamp. |  |
| e) | Give requirements of arc welding source. |  |
| f) | Draw speed-time curve for urban and suburban electric traction services. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List and explain different modes of heat transfer. Give expression of heat transfer in each mode. |  |
| b) | Give any four requirements of good heating element. Name two materials used for heating element. |  |
| c) | A 30KW, 3 phase, 400 Volts resistance oven uses Nickel-Chrome strip 0.025cm thick for the 3 phase star connected heating element. If the wire temperature is to be 1100C and that of the charge is to be 700C, calculate the suitable length and width of the heating element. Given Radiating efficiency = 0.6, Emissivity = 0.9, specific resistance of Nickel-Chrome = 1.03 x 10-6 m. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List different types of arc furnaces. Explain working of any one type with a neat diagram. Give its applications. |  |
| b) | With a neat diagram explain construction and working of coreless induction furnace. |  |
| c) | Give advantages and disadvantages of eddy current heating. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of arc blow. List different methods of reducing arc blow. |  |
| b) | With a neat diagram explain Flash welding. Give any two applications of the same. |  |
| c) | Give advantages & disadvantages of electric traction system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain typical speed-time curve for Main line traction services. |  |
| b) | Give required features of traction motors. |  |
| c) | With a neat diagram explain shunt transition method. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define following terms: (i) Illumination (ii) Mean horizontal candle power (iii) Utilization factor. | ..…2/- |
|  | -2- |  |
| b) | Explain the concept of stroboscopic effect. Explain any two methods of reducing stroboscopic effect. |  |
| c) | With a neat diagram explain construction and working of high pressure Mercury Vapour lamp. |  |

***********