## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg./FT&EE

Subject: Elements of Civil Engineering(3003) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Stating their suitability name different types of piles used in building construction. |  |
| b) | Listing their merits, explain where do you use following types of roof coverings: - Tiles ii) Acrylic sheets iii) A.C sheets |  |
| c) | Explain how do you perform temporary adjustments of a levelling instrument. |  |
| d) | Stating their merits list out various materials used for acoustic purpose in an auditorium. |  |
| e) | What are the precautions to be taken in compacting the concrete using vibrators? |  |
| f) | What are the methods adopted in transporting the concrete? |  |
| g) | Explain the procedure involved in manual mixing of concrete. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure involved to overcome obstacle of a river in chain surveying. |  |
| b) | With an example for each, explain direct and indirect method of ranging a survey line. |  |
| c) | The following bearings were observed in a traverse survey. Side Fore bearing Back bearing PQ 1100' 2900' QR 25 0' 2050' RS 22030' 4030' ST 18730' 730' TP 1250' 3050' Calculate the interior angles of the traverse. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Stating their merits, explain rise and fall method and height of instrument method. |  |
| b) | The following consecutive readings were taken with a levelling instrument at regular intervals 2.150, 1.860, 0.720, 2.890, 2.050, 1.780, 1.350, 0.770, 0.540, 1.900, 2.250, 2.990 The instrument was shifted after 4th and 8th readings. The last reading was taken on a bench mark of RL 101.000 m. Find the RLs of all the points. |  |
| c) | i) Describe procedure, how reciprocal levelling is carried out. ii) Define the following terms: i) Axis of telescope ii) Change point iii) Line of collimation |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure involved in measuring the horizontal angle using the theodolite. | ……2/- |
|  | -2- |  |
| b) | Explain how do you verify the verticality of a tower using a theodolite. |  |
| c) | What do you understand by thermal insulation? Stating their suitability, explain various materials used for thermal insulation. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Stating the situations where they are used, describe the construction of pile foundations. |  |
| b) | Describe the type of flooring systems used for following structures: - Workshop building ii) Auditorium |  |
| c) | With the aid of neat sketch explain the construction of North light roof truss. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | What do you understand by Reverberation? State how do you control the same. OR | (3) |
| a) | Draw conventional signs used in building drawing to represent the following: - Electric switch - Stone masonry - Glazing window | (3) |
| b) | The following are the construction details of a power house: - Size of room 4.0m x 5.50m. - Thickness of stone masonry wall all around-300mm - Plinth height above GL- 500mm - Roof-RCC slab 120mm thick, 250mm projecting all round the room - Door 1.2m x 2.1m. - Window 1.3 x 1.5m - Height of roof above floor – 3.1m Draw - Plan ii) Section passing through window | (3) (6) |

***********