## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg.

Subject: Power Plant Engineering(5037)/(ME623) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is mean by load curve? What is its significances in power generation? |  |
| b) | Give the advantages and disadvantages of hydroelectric power plant. |  |
| c) | List the impurities that are present in water. What are their acceptable limits if used for boiler feed water? |  |
| d) | What are the advantages of burning the coal in pulverized form? |  |
| e) | What are the desirable properties of a good coolant used in nuclear rectors? |  |
| f) | How intercooling improves the thermal efficiency of a simple open cycle gas turbine power plant? |  |
| g) | What do you understand by carry over losses? State the different method used to reduce these losses. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The maximum load on a thermal power plant of 80 MW capacity is 65 MW at an annual load factor of 70%. The coal consumption is 1.5 kg per unit of energy generated in KWh and the cost of coal is Rs.600 per ton of coal. Find i) Annual revenue earned if the energy is sold at Rs.2 per KWh ii) Energy generated per year. |  |
| b) | What are the factors to be considered while selecting a site for hydro power plant? |  |
| c) | Explain with neat sketches following systems of an Internal combustion engine power plant: - Air intake system (ii) Exhaust system |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the working of a closed cycle gas turbine power plant. |  |
| b) | What factors are to be considered in designing a coal handling plant? |  |
| c) | With a neat sketch explain the construction and working principle of cyclone dust collector. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the De-aeration process used in water treatment. |  |
| b) | With the help of neat sketch, explain the construction and working of induced draft cross flow type cooling tower. |  |
| c) | What are the functions of gas loop in thermal power plant? Why is necessitates cleaning of flue gas? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of coal preparation plant in Thermal power station & explain in brief. |  |
| b) | Explain demineralization process of water treatment system with neat sketch. |  |
| c) | Explain with neat sketch construction and working of “Pressurized Water Reactor” used in nuclear power plant. | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various methods of radioactive waste disposal. |  |
| b) | Draw the neat sketch of a hyperbolic cooling tower & explain its construction & working. |  |
| c) | What important factors are considered in selecting a site for thermal power plant? Give some advantages of thermal power plant. |  |
| d) | Draw a neat line diagram of a diesel power plant showing all the systems. |  |

***********