## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Medical Electronics

Subject: Electronic Devices & Circuits(MX401)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat circuit diagram of voltage divider biasing circuit. State its one advantage. |  |
| b) | What is negative feedback? State its two advantages. |  |
| c) | Write equation of frequency of oscillation for Hartley oscillator. Draw circuit diagram of Hartley oscillator. |  |
| d) | State any three types of negative feedback in amplifiers. |  |
| e) | Draw symbol of UJT, SCR and DIAC. |  |
| f) | State any three applications of S.C.R. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With a neat circuit diagram, explain collector to base bias circuit. |  |
| b) | Compare RC coupled and Transformer coupled multistage Amplifier. (any 6 points) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat labelled circuit diagram explain RC coupled multistage Amplifier. |  |
| b) | Draw a neat circuit diagram of single stage CE Amplifier and state the function of each component in the diagram. |  |
| c) | Compare ‘Positive feedback’ and ‘Negative feedback’. (any 6 points) |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With a neat circuit diagram, explain the working of Tuned Collector oscillator. |  |
| b) | With a neat circuit diagram, explain the working of Colpitt’s oscillator. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain the construction of S.C.R. and also draw its V-I characteristics. |  |
| b) | Explain working of Diac with a neat V-I characteristics. |  |
| c) | Explain gate triggering method of SCR. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat circuit diagram and waveforms, explain the operation of a single phase half wave controlled rectifier with resistive load. |  |
| b) | With a neat circuit diagram and waveforms, explain the working of single phase half wave controlled rectifier with R-L load. |  |
| c) | With neat diagram and waveforms, explain the operation of a single phase bridge type inverter. |  |

***********