## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-II(4008)/(CE401) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Transiting the telescope ii) Face right iii) Latitude. |  |
| b) | Write down any three characteristics of Tacheometer. |  |
| c) | Draw the diagram for Valley line, Summit, Vertical cliff. |  |
| d) | Draw figures of any three types of curves. |  |
| e) | What is vertical curve & why it is provided? |  |
| f) | State the application of GPS. (only for students of revised scheme) |  |
|  | Only for students of old scheme: |  |
| f) | What is orientation of plane table? Explain any one. |  |
| g) | List out the steps for final location survey for roads. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following records are obtained in a traverse survey where the length and bearing of the last line were not recorded. Calculate the length and bearing of line DA. Line; Length (m); Bearing. AB; 75.50; 30 24. BC; 180.50; 110 36. CD; 60.25; 210 30. DA; ?; ?. |  |
| b) | Explain in detail the procedure of reiteration method of measuring horizontal angles with neat sketch. |  |
| c) | What are the temporary adjustments in theodolite? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any six parts of theodolite with their function. |  |
| b) | Explain the procedure for measurement of magnetic bearing of a line by using theodolite. |  |
| c) | Two tangents AB and BC intersect at a point B at chainage 180.5m. Calculate all necessary data for setting out a circular curve of radius 100m and deflection angle 30 by the method of offsets from the long chord. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the principle of tacheometry with neat sketch. |  |
| b) | Determine the values of stadia constants from the following observations. Instrument station; Staff reading (m) on; Dist (m); Stadia reading. Lower; Upper. 0; A; 155; 1,255; 2.750. B; 250; 1.000; 3.000. C; 300; 0.750; 3.255. |  |
| c) | Explain the methods of setting a simple circular curve by taking offset from the long chord. (only for students of revised scheme) -2- | …….2/- |
| c) | Describe intersection method of plane table survey. (only for students of old scheme) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of sketch explain indirect method of using square for locating a contour. |  |
| b) | Explain arithmetical method of interpolation of contour. |  |
| c) | Explain the methods of setting a simple circular curve by Rankine’s deflection method. (only for students of revised scheme) |  |
| c) | Explain preliminary survey for roads (only for student of old scheme) |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write a short note on Gale’s traverse table. |  |
| b) | Write down any four characteristics of tachometer. |  |
| c) | Define i) Contour line ii) Contour interval iii) Horizontal equivalent iv) Contour gradient. |  |
|  | only for students of revised scheme: |  |
| d) | Why total station are preferred in modern day large construction projects? Give various applications of total station. |  |
| e) | Write short note on Digital level. |  |
|  | only for students of old scheme: |  |
| d) | List down four accessories of plane table survey and state its uses. |  |
| e) | What is Reconnaissance survey? |  |

***********