## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg.(CT)

Subject: Construction Structure Design(CT702)

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 456-2000, IS 8000-2007 & Steel Tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List out various loads and their combinations considered in design of structures. |  |
| b) | What do you understand by partial safety factors? State the values for materials and loads you take in design of RC members. |  |
| c) | With the aid of strain diagram explain what you understand by ‘Balanced’, ‘Under reinforced’ and ‘Over-reinforced’ sections. |  |
| d) | What do you understand by ‘State beam theory’. Explain its application in RC member design. |  |
| e) | Explain design procedure of cantilever retaining wall. |  |
| f) | What are the merits and demerits of welded connections over rivetted connections? |  |
| g) | Sketch various types of structural steel sections and state fields of applications of each section. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Show that critical neutral axis depth for an RC section is 0.53d, 0.48d and 0.46d with Fe250, Fe415 and Fe500 steels respectively. |  |
| b) | Taking the width of the beam equal to half the depth, determine the dimension of beam and tensile reinforcement required for an RC beam. The udl at service condition on beam is 25KN/m and effective span of beam is 4m. Take M20 concrete and Fe415 steel. |  |
| c) | What do you understand by development length? Calculate development length for following cases. Concrete; M20; M25; M30. Steel; Fe250; Fe415; Fe500. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A singly reinforced beam 250mm x 400mm (effective) is reinforced with 3#16 bars. If M20 concrete and Fe415 steel is used, determine the safe working load the beam can carry at working condition over an effective span of 4.5m. |  |
| b) | Design a slab for a room of 4m x 5m as clear dimension. The slab is supported on 230mm wide beams alround. Two adjacent edges of slabs continuous and remaining two edges are discontinuous. Use M20 concrete, Fe415 steel. Take LL=2.5KN/m2, FF=1KN/m2. |  |
| c) | An RC beam simply supported over an effective span of 5m, carries an udl of 30 KN/m at service condition. The beam is 250mm x 400mm (effective) with 3#12mm as tensile reinforcement. Design shear reinforcement with M25 concrete and Fe415 steel. |  |
|  | -2- | ……2/- |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following: i) Doubly reinforced sections and situations where they are adopted. ii) Requirement of minimum eccentricity for RC column sections. iii) Curtailment of stem reinforcement in cantilever retaining wall. |  |
| b) | Design a circular column section with lateral ties to carry an axial load of 500KN at service condition. The effective length of column is 4m. Use M25 concrete and Fe415 steel. |  |
| c) | Design a square footing to carry an axial column load of 750KN at service condition. The column is 300mm x 300mm. The bearing capacity of soil is 160KN/m2. Use M20 concrete and Fe415 steel. Carryout necessary checks for the footing designed. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An ISMB 400 @ 604.3 N/m is used as beam over an effective simply supported span of 4.5m. Determine the udl at service condition the beam can support over entire span. The steel is Fe410 with fy = 250MPa. |  |
| b) | Classify the following sections taking fy = 250MPa i) ISA 90 x 60 x 8 @ 87.3 N/m ii) ISLC 150 @ 141.3 N/m iii) ISLB 500 @ 735.6 N/m |  |
| c) | Design an angle purlin section provided over trusses provided at 4m c/c. The panel length of purlin is 1.4m. The roof covering weighs 160 N/m2 and intensity of wind load is 1.2 KN/m2. The slope of roof is 26°. Take fy = 250MPa. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A channel section ISMC 125 @ 124.6 N/m is connected at its end by 4mm size weld with its web to a gusset plate of 8mm thick. The length of weld on top and bottom in longitudinal directions are 125mm each and 125mm length in transverse direction. Determine the tensile strength of the member. Take fy = 250MPa. |  |
| b) | Design a double angle discontinuous strut to carry factored axial load of 150 KN. The length of strut between centre to centre of intersection is 2.5m. Take fy = 250MPa. |  |
| c) | Design slab base with suitable welded connection to carry an axial load of 500 KN at service condition. The slab base is resting on concrete pedestal of M20 concrete with bearing strength of 0.45 fck. Take fy = 250MPa. |  |

***********