## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./FT&EE

Subject: Elements of Civil Engineering(3003)/(CS311) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between direct and indirect ranging. |  |
| b) | What is local attraction? How do you detect it in compass survey? |  |
| c) | How do you carryout temporary adjustments needed for each new setting of leveling instrument? |  |
| d) | List out various types of roof coverings used and state their suitability. |  |
| e) | State suitable floors for following structures: i) Factory building ii) Auditorium iii) Living room |  |
| f) | Draw labelled sketch of north light roof. |  |
| g) | Sketch conventional signs used to represent the following in a building drawing: i) Electric fan ii) Wash basin iii) Door opening |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat sketch explain procedure for direct ranging. |  |
| b) | The following are the fore bearings observed in a closed traverse. Calculate the interior angles of the traverse. Line; Fore bearing. AB; 60°45. BC; 125°30. CD; 210°15. DA; 300°0. |  |
| c) | Explain different parts and their functions of dumpy level. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following consecutive readings were taken with a levelling instrument at 15m intervals. 2.010, 1.820, 0.850, 3.350, 2.400, 2.135, 1.555, 0.955, 1.565, 1.690, 2.300 and 3.630. Instrument is shifted after 4th and 8th reading. Calculate RL of points by Rise and Fall method. RL of first point is 100.5m. |  |
| b) | Write the procedure to measure vertical angle by using theodolite. |  |
| c) | With neat sketch explain the method for prolonging a line by using theodolite. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Name the type of flooring used for factories and explain the procedure involved in construction of it. |  |
| b) | Differentiate between shallow foundation and raft foundation. -2- | …..2/- |
| c) | Draw neat labelled sketch of Grillage foundation and explain suitability of such type of foundations. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between following: i) Floating piles and End-bearing piles ii) Cast in situ piles and Precast piles |  |
| b) | Write short notes on: i) Compaction ii) Curing |  |
| c) | Write the measures taken for fire protection in multi-storied buildings. (only for students of revised scheme) |  |
| c) | Describe various insulting materials used for thermal and acoustic insulation of buildings and state their suitability. students of old scheme) | (only for |
| Q.No.6. | The following are the constructional details of a single room with attached toilet. Size of room 3.4m x 4.5m Attached toilet 1.20m x 1.6m Height of the roof above floor 3.2m Height of plinth 0.5m Doors 1.0m x 2.10m Windows 1.25m x 1.50m Ventilators 0.6m x 0.6m Parapet wall 0.20m thick and 0.9m height above roof Wall thickness 0.23m Roof- RCC slab 120mm thick Chajja projection 0.6m from face of wall Roof projection alround 0.3m Draw the plan and section passing through the window opening. Use 1:50 scale. | (12) |

***********