## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg./A

Subject: Thermal Engineering(ME303) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Mollier’s chart, Steam Tables, ISBN-81-224-0644-0

is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between extensive and intensive properties. Give example of each and state their units. |  |
| b) | Define potential energy and state its units. |  |
| c) | State and explain Zeroth’s law of Thermodynamics. |  |
| d) | State and explain Gay-Lussac’s Law. |  |
| e) | Define Mach number and state its significance. |  |
| f) | State and explain Dalton’s Law of partial pressures. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An airconditioner has to maintain a temperature of 10ºC in the cooled space. If the ambient atmospheric condition has a temperature of 37ºC. Find the Carnot COP of the air conditioner. |  |
| b) | Determine the mass of nitrogen in a cylinder of 0.5m3 capacity at a pressure of 150 bar and a temperature of 23ºC. Take universal gas constant as 8.314 KJ/Kg mol ºK |  |
| c) | A certain quantity of air has a volume of 0.048m3 at a pressure of 1.5 bar and 37ºC. It is compressed to a volume of 0.0024m3 according to the law PV1.3 = constant. Find the work done during compression and the final temperature reached. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a Rankine cycle, the maximum pressure of steam supplied is 6 bar and its dryness fraction is 0.96. The exhaust pressure is 0.8 bar. Using Mollier’s chart, find Rankine efficiency. |  |
| b) | Calculate the internal energy of 1 Kg of steam at a pressure of 9.8 bar when - The steam is 0.6 dry - The steam is superheated and the temperature of steam is 260ºC Take sp. heat of superheated steam at constant pressure as 2.2 KJ/KgK. |  |
| c) | Explain construction and working of a Benson boiler with the help of a neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the construction and working of a Cornish boiler. |  |
| b) | What are the functions of blow off cock? Explain its construction and working with help of a neat sketch. |  |
| c) | Explain construction and working of an impulse type of a steam turbine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain regenerative feed heating in steam turbine. -2- | …….2/- |
| b) | Explain construction and working of a induced draught cooling with help of a neat sketch. |  |
| c) | What are the functions of steam condenser? Explain how condensers are classified. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the process of isobaric expansion. Represent the process on P-V and T-S diagram. Write an expression for the work done during the process. |  |
| b) | Explain the equivalence of Kelvin-Planck and Clausius statements of the second law of thermodynamics. |  |
| c) | With the help of a neat sketch explain the construction and working of a steam colorimeter. |  |

***********