## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mining Engg.

Subject: Opencast Mining Machinery(4299)/(MN501) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the various fishing tools used during drilling? |  |
| b) | What are the different elements of Mechanical transmission? |  |
| c) | Compare crawler mounted shovel with a wheel mounted shovel. |  |
| d) | List down various safety devices in dumpers. |  |
| e) | Mention the various components of a shank in ripper. |  |
| f) | Explain the purpose of Air vessels in pumps. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the components & their purpose of percussive drilling machine used in opencast mines. |  |
| b) | What are the different types of core barrels? How they are different from each? |  |
| c) | Explain the construction & working of a bull dozer with sketches. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and operation of a dragline bucket with a neat sketch. |  |
| b) | Write short notes on: i) Swing mechanism ii) Bucket system of a hydraulic shovel. |  |
| c) | Explain the construction & operation of the bucket wheel excavator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the mechanical transmission system in a dumper. |  |
| b) | i) What are the different types of belts in a belt conveyor? ii) What is the purpose of loop take up arrangement? Draw a simple sketch. |  |
| c) | Write short notes on: i) Ropes ii) Trestles in a bi-cable aerial rope way. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch showing the components of a centrifugal pump. What is the main function of each component? |  |
| b) | i) What is an axial end thrust? How do you overcome it? ii) How do you prevent cavitation in a pump? |  |
| c) | i) Write short notes on ‘Characteristic curves of a pump’. ii) Enumerate the factors on which selection of a pump is done. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is bore-hole deviation? Mention the causes. |  |
| b) | Explain the operation of a clamshell. |  |
| c) | What are the different types of dumpers used based on the construction? |  |
| d) | Explain the salient features of milling drum type surface miner. |  |

***********