## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mining Engg.

Subject: Underground Mining Methods(MN402) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Classify the coal seam on the basis of thickness. |  |
| b) | Enumerate advantages of Long wall method. |  |
| c) | Write application conditions for Blasting gallery method. |  |
| d) | Explain over hand and under hand stoping. |  |
| e) | Compare cut and fill method with shrinkage method. |  |
| f) | Explain cavability index. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain systematic support rule. |  |
| b) | With sketch explain method of depillaring for strong coal and strong roof/ weak roof strong coal. |  |
| c) | The following information is given on depillaring operation: Coal seam height–2.1m, depth-200m, pillar size-30m x 30m, Gallery size- 4m x 2.1m, specific gravity of coal 1.4, Advance of split/blast–1m, No. of split–4, blast of split per shift – 2, working shift in a day-3. Calculate production from district. Assume other conditions. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain cyclic working of Long wall. |  |
| b) | Explain hydraulic support/AFC. |  |
| c) | With sketch explain construction of shearer. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain thick seam working by sub level caving method. |  |
|  | OR |  |
| a) | Explain induced caving in Blasting gallery method. |  |
| b) | With sketch explain Blasting gallery method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enumerate advantages and disadvantages of sub level caving method. |  |
| b) | List out suitability conditions for square set stoping and explain elements of set. |  |
| c) | With sketch explain Long hole stoping. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Block caving method |  |
| b) | Cycle of operation in sublevel caving method |  |
| c) | Blast hole stoping method |  |
| d) | Development of stope |  |

***********