## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Computer Engg.

Subject: Embedded Systems Design(CO614)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is an Embedded System? Give any two examples of Embedded Systems. |  |
| b) | What is memory shadowing? |  |
| c) | What is the role of Real Time Clock in an Embedded System? |  |
| d) | Explain: (i) Context Switching (ii) Context Saving. |  |
| e) | What is a Semaphore? List different types of Semaphores. |  |
| f) | Give any three differences between Microprocessors and Microcontrollers. |  |
| g) | What are Hard and Soft Real Time Systems? Give examples. |  |
| h) | What is the role of Device drivers in the operating system context? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give comparisons between Embedded systems and General computing systems. |  |
| b) | Explain any six characteristics of an Embedded system. |  |
| c) | What is non-operational quality attribute? Explain the important non-operational quality attributes to be considered in Embedded system design. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following types of ROM memories in an Embedded system: (i) Masked ROM (ii) PROM (iii) EPROM. |  |
| b) | With the help of a neat diagram explain the components of a typical Embedded system. |  |
| c) | Briefly explain any three External Communication Interfaces. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Reset circuit and Brown out protection circuit in an Embedded system. |  |
| b) | Explain the Assembly Language based Embedded Firmware Development. |  |
| c) | Explain the various Embedded Firmware design approaches in detail. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a Real Time Kernel? Explain the basic functions of Real Time Kernel. |  |
| b) | Briefly explain any three non-preemptive task scheduling algorithms. State its merits and demerits. |  |
| c) | What is a Thread? Explain the concept of Multithreading in detail. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a Deadlock? Explain the different conditions favouring Deadlocks. |  |
| b) | With the help of a neat diagram explain Remote Procedure Call (RPC) for Inter Process Communication. |  |
|  | -2- |  |
| c) | Briefly explain the various functional and Non functional requirements to be evaluated in the selection of an RTOS. |  |

***********