## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE

Subject: Switchgear & Protection(4110)/(EE501) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List any six causes of fault in a power system. |  |
| b) | Explain the concept of current limiting reactor in brief. |  |
| c) | For a fuse define i) Fusing factor ii) Pre-arcing time iii) Breaking capacity. |  |
| d) | Draw a neat diagram of HRC fuse and name its parts. |  |
| e) | State advantages of circuit breaker over a fuse. |  |
| f) | For a relay define i) Relay ii) Pick-up iii) Reset. |  |
| g) | Explain the classification of relays. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 3 phase transmission line operating at 10 KV and having a reactance of 4 is connected to the generating station bus-bars through 5MVA step-up transformer having a reactance of 5%. The bus-bars are supplied by a 10MVA alternator having 10% reactance. Calculate the short circuit KVA fed to the fault if it occurs at end of transmission line. |  |
| b) | State six points of comparison between Re-wirable fuse and HRC fuse. |  |
| c) | State and explain the methods of arc extinction in circuit breakers. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain working principles and construction of MCB. |  |
| b) | With a neat diagram explain arc extinction in SF6 circuit breaker using puffer piston arrangement. |  |
| c) | Write a note on HVDC circuit breaker. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of bimetallic relay. |  |
| b) | Describe the working of induction type overcurrent relay. (bi-directional) |  |
| c) | With a neat block diagram, explain the working of static overcurrent relays. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain biased differential protection for transformers. |  |
| b) | Explain the current graded scheme of protection for transmission line. |  |
| c) | With a neat diagram explain earth leakage protection for bus-bar protection. |  |
| d) | What is the cause of unbalanced supply voltage in motor? |  |
|  | Also state its effects. (only for students of old scheme) |  |
|  | -2- | ……2/- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram explain the protective scheme used in protection of three phase induction motor against single phasing. |  |
| b) | Explain in brief the following types of air blast circuit breakers i) axial blast type ii) cross blast type. |  |
| c) | Write a short note on MCCB. |  |
| d) | Describe bus bar protection scheme for double bus bar system. (only for students of old scheme) |  |

***********